## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain principles of surveying. |  |
| b) | Give conventional symbols for the following: - Embankment - River - Wire fencing. |  |
| c) | Define - Bench Mark - Datum - Change point. |  |
| d) | Convert following WCB to QB - 179 30 - 90 - 320. |  |
| e) | Explain the terms - Check line - Oblique offset - Base line. |  |
| f) | List the instruments commonly used for Direct leveling & state purpose of each instrument. |  |
| g) | Explain the classification of survey based on instrument. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of stepping in chaining on sloping ground with the help of neat sketch. |  |
| b) | The distance between two stations was 1500 m, when measured with 20 m chain. The same distance when measured with 30 m chain was found to be 1476m. If 20m chain was 5 cm too short, what was the error in 30m chain? |  |
| c) | Explain the procedure for plotting of traverse. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Below are the bearing observed in traverse survey conducted by using prismatic compass Line; FB; BB. PQ QR RS SP; 124 30 68 15 310 30 200 15; 304 30 246 135 15 17 45. |  |
|  | At what station do you suspect local attraction? Find corrected bearings & included angle. |  |
| b) | How will you continue chaining past a tall building? |  |
| c) | List down instruments used for setting perpendicular. Explain in detail principle & working of any one of these instruments. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with sketch procedure of profile levelling. |  |
| b) | Calculate area ABCDEFA of cross staff survey of field |  |
| c) | - Why is zero marked on south pole of prismatic compass? - Show by neat diagram graduations on ring of prismatic compass. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain two methods of calculation of reduced level & write comparison between two. |  |
| b) | Following readings were taken with dumpy level & 4m levelling staff on continuously sloping ground. Calculate RL of different points if RL of BM = 100 & readings are 0.905, 1.745, 2.345, 3.125, 3.725, 0.545. 1.39, 2.055, 2.955, 3.455, 0.595, 1.015, 1.85, 2.655, 2.945. |  |
| c) | Explain sources of errors in levelling. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | A 30m chain |  |
| b) | BM & its type |  |
| c) | Traversing by chain/compass |  |
| d) | Abney level |  |
| e) | Local attraction |  |
| f) | Levelling staff |  |

***********