## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Mining Engg.

Subject: Mine Development(4293)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| I) | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| 1) | Give the salient features of IBM. |  |
| 2) | Explain the terms: (a) Seam (b) Ore. |  |
| 3) | What advantages does circular shaft offer compared to a rectangular shaft? |  |
| 4) | Define the different stripping ratios. |  |
| 5) | What are the advantages & disadvantage of Bord & Pillar mining? |  |
| 6) | Write short notes on subsidence. |  |
| II) | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain the different stages of mining operations. |  |
| 2) | Write short notes on: (a) beneficiation (b) prospecting. |  |
| 3) | What is box cut? How is it constructed? |  |
| III) | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | With neat sketches explain the modes of opening a steeply inclined deposit. |  |
| 2) | Draw a cross-section of an opencast mine & indicate different elements. |  |
| 3) | Draw a neat sketch of a metal mine block/stope. Explain the different parts. |  |
| IV) | Answer the following Questions: |  |
| 1) | A district is being developed by Bord & Pillar and the gallery size is 3.5mx3m. Calculate the quantity of explosive used and production per blast from each face. Assume any data if required. | (8) |
| 2) 2) | Draw a neat sketch of development panel showing transportation system by conveyor belt. ### OR Differentiate between solid blasting & blasting by coal cutting machine. | (4) (4) |
| V) | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain the steps that need to be taken to prevent the failure of slopes. |  |
| 2) | What is the angle of draw? What are the factors affecting it? |  |
| 3) | Explain the terms: (a) Modulus of rigidity (b) Poisson’s ratio. |  |
| VI) | Answer any three of the following Questions: | 3 x 4 = 12 |
| 1) | Explain circular failure of slope with a neat sketch. |  |
| 2) | Write short notes on Adit mode of entry. |  |
| 3) | What is Level Interval? What are the factors affecting Level Interval? |  |
| 4) | - Write short notes on Reserves of iron ore in Goa. - Give the advantages of an opencast mine. |  |

***********