## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-II(4144)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write short note on Fading. |  |
| b) | Define standing wave ratio & reflection coefficient. |  |
| c) | Explain the construction of loop antenna & state its applications. |  |
| d) | State the advantages of waveguides over coaxial cables. |  |
| e) | Explain aliasing effect in PAM. |  |
| f) | State the advantages of TDM over FDM. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Classify various modes of radio wave propagation. Explain any one in detail. |  |
| b) | Explain with the help of simple diagrams, the phenomenon of super refraction or duct propagation. |  |
| c) | Draw & Explain the standing waves on transmission line for following load impedance i) ZL=   ii) ZL = 0 . |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Differentiate between resonant & non-resonant antenna. Draw & explain the radiation pattern of various length resonant dipoles. |  |
| b) | Explain the Construction of radiation pattern of Rhombic antenna. |  |
| c) | With a neat diagram, explain TE1,0 mode of operation of a waveguide. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Define PAM. Explain the technique of generating a PAM signal. |  |
| b) | With neat waveform, explain PWM. Explain PWM detection technique. |  |
| c) | With the help of block diagram, explain the operation of PCM transmitter. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the operation of ADM receiver, with neat block diagram. |  |
| b) | With neat diagram & waveform, explain FSK generation technique. |  |
| c) | Explain QPSK receiver operation using neat block diagram. |  |
| Q.No.6. | Write short noted on any three: | 3 x 4 = 12 |
| a) | FDM |  |
| b) | Slope overload error & granular noise |  |
| c) | ASK |  |
| d) | Stub matching |  |

***********