## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Basic Instrumentation-I(S-4214)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| 1) | Explain the use of measurement systems in experimental engineering analysis. |  |
| 2) | Describe the classification of transducers in brief. |  |
| 3) | With a diagram, explain the working principle of RTD. |  |
| 4) | Draw and label neat diagrams of bonded and unbonded strain gauge. |  |
| 5) | Write an expression of capacitance of a parallel plate capacitor and state various methods of changing the capacitance. |  |
| 6) | On what principle does the moving magnet type electromagnetic transducer work? |  |
| 7) | Draw input V/s output characteristic of LVDT and give reason for the presence of small output voltage at null position. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Define “Calibration”. List various calibration procedures and explain any two briefly. |  |
| 2) | Differentiate between (i) Null type and deflection type Instruments (ii) Basic functional elements and auxiliary elements. (Give atleast 3 points of difference for each) |  |
| 3) | State the principle and briefly explain the working of any two self generating type transducer elements with diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Explain two temperature compensation techniques used with a resistance strain gauge in a bridge circuit. |  |
| 2) | Compare (in four points) the performance of the following temperature sensors: RTD, Thermistor and IC sensor. |  |
| 3) | Draw and explain the schematic diagram of temperature measurement in a tank using platinum resistance as a transducer. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | Draw and explain displacement measuring system using LDR. |  |
| 2) | Draw a neat sketch of electrodynamic and magnetostrictive transducer and briefly explain their working. |  |
| 3) | Explain the single and differential output measurement arrangements used with Inductive transducers. |  |

………2/-

-2-

| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| 1) | State four typical specifications, advantages, disadvantages and applications of LVDT. |  |
| 2) | Explain the operation of proximity type Inductance transducers with the help of a diagram. |  |
| 3) | Draw the following characteristics of a capacitance: - Area v/s Capacitance - Relative Permittivity v/s Capacitance - Distance v/s Capacitance. |  |
| 4) | List 8 desirable characteristics of a transducer element. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| 1) | With a neat sketch, explain the measurement of angular displacement using capacitive transducer. |  |
| 2) | Draw a block diagram of displacement measuring system using a capacitive pickup and explain its working. Also draw output waveforms at each stage. |  |
| 3) | Draw and explain a schematic of a capacitive transducer to measure a linear displacement with differential output. State advantages of differential output over single output. |  |

***********