## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg/CT

Subject: Analysis of Structures(4026)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Define Limit of Eccentricity. Also derive expression for same for a rectangular column carrying an eccentric load for no tension condition. |  |
| b) | A column is subjected to direct compressive stress of 85 N/mm2. Determine normal, tangential stress and resultant stress on a plane of 45º to axis of column. |  |
| c) | Using Moment–Area theorem derive expression for maximum slope and deflection for a simply supported beam with UDL throughout the span. |  |
| d) | Define following terms in Moment Distribution Analysis: - Relative Stiffness 3. Carry Over Factor - Distribution factor 4. Total Stiffness. |  |
| e) | Derive expression for fixed end moments for a fixed beam of span ‘L’ carrying point load at centre. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A wooden beam 150mm wide, 250mm deep consists of steel plates of 12mm thick and 200mm deep connected to sides of wooden beam symmetrically. If the beam is simply supported for a span of 6m with UDL of 20KN/m throughout span, determine stresses produced in steel and wood. Take Es = 18Ew. |  |
| b) | A steel plate of 10mm thick and 150mm deep is connected between the wooden beam of 125mm wide and 220mm deep. If stress in steel and wood are not to exceed 8 N/mm2 and 100 N/mm2 respectively, determine moment of resistance of flitched beam. Take Es=15Ew. |  |
| c) | A hollow circular column of 200mm external diameter and 160mm internal diameter carry axial load of 80KN. A bracket connected to column transfers an eccentric load of 220KN at a distance of 300mm from axis of column. Determine maximum and minimum stresses produced in column. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a certain point of elastic material, a direct tensile stress of 60N/mm2 and direct compressive stress of 40N/mm2 acts at right angles to each other. If maximum principal stress is limited to 75N/mm2 tensile, find shear stress that may be allowed on the planes. Also find minimum principal stress and maximum shear stress. |  |
| b) | In a shaft transmitting power shearing stress at surface of shaft is 60N/mm2 and a bending tensile stress of 90N/mm2. Determine normal, tangential and resultant stress on a plane marking an angle of 60º with tensile stress. |  |

………2/-

-2-

| c) | At a point in a material, principal stresses of 140N/mm2 and 80N/mm2, both tensile, acts at right angles to each other. Locate the plane at which resultant stress has maximum obliquity. Also find Normal stress and tangential stress. Use Mohr’s Circle method. |  |
| --- | --- | --- |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever beam of span 3m carry an UDL of 10 KN/m throughout span along with point loads of 5 KN at centre of span and 2KN at 1m from free end. Take EI=8x1012 N.mm2. Determine maximum slope and deflection in beam. Use Macalay’s method. |  |
| b) | A fixed beam of span 5m carry concentrated loads of 12KN, 18KN and 6KN at a distances of 2m, 3m and 4m from left support. Determine Fixed End moments and draw BMD of beam. |  |
| c) | A fixed beam of span 6m carries an UDL of 20KN/m for a length of 4m from left support. Find Fixed End Moments and draw SFD of beam. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A continuous beam ABCD has AB=4m, BC=5m and CD=4m. End A is fixed where as all other supports are simple supports. EI of AB member is twice of that of BC and CD members. AB carries an UDL of 30KN/m, while BC carries point load of 40 KN at centre and span CD carries a point load of 15KN at 1m from right end D. Analyse beam by Moment Distribution method and draw SFD and BMD. |  |
| b) | A portal frame ABCD has fixed supports at A and B. Beam BC carries UDL of 40 KN/m throughout along with point load of 20KN at centre. Flexural rigidity of AB and CD is 1.5 EI while BC is 2EI. Analyse frame for support moments and draw BMD. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam of 5m span carries points loads of 4KN at 2m and 30KN at 4m from left support. Find deflection under both loads. Take EI=12x1012 N.mm2. |  |
| b) | A hollow circular column of 3m long has one end fixed and other end hinged. It has to support axial load of 500KN. Allowing factor of safety of 4, determine dimensions of column. Take internal diameter of column 0.8 times external diameter. Use Euler’s formula. E=2x105 N/mm2. |  |
| c) | A column of timber section 160mm wide and 200mm deep is 6m long with both ends fixed. Find safe load for column using factor of safety of 3. Take crushing stress as 50N/mm2 and Rankine’s Constant = 1/750. |  |

***********