## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Structural Drawing(4341)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Use of Bonjean curves |  |
| b) | Deck stringer and shear strake |  |
| c) | Neutral axis and section modulus |  |
| d) | Use of shell expansion plan |  |
| Q.No.2. | A box shaped vessel 100m long floats on an even keel displacing 2000t. The mass of the vessel alone is 1000t evenly distributed and it is loaded at each end of the length of 25m with 500t of cargo also evenly distributed. Sketch the curve of load shearing force and bending moments. Also state the maximum shearing force and bending moment and state where these occur. | (12) |
| Q.No.3. | From fig.(01), find | (12) |
|  | 1.The position of neutral axis |  |
|  | 2.Moment of Inertia of the total area of the material about the neutral axis |  |
|  | 3.Section Modulus about the deck and bottom. |  |
| Q.No.4. | Answer the following: |  |
| a) | Briefly describe, how a shell expansion drawing is obtained using ships lines plan. | (9) |
| b) | How numbering of plates is done in shell expansion plan? | (3) |
| Q.No.5. | Answer the following: |  |
| a) | Explain briefly by taking example of a box shaped vessel how the section modulus of the ship can be increased by adding minimum area to the cross section. | (8) |
| b) | Explain to what extent the section modulus of a ship is increased and what is safety factor. | (4) ….2/- |
|  | -2- |  |
| Q.No.6. | Draw a mid-ship section of a bulk carrier and show details of collar plate, scallops cross section I.W.O. stiffeners and also attachment of vertical stiffeners with lower and upper hoper. | (12) |
| Q.No.7. | Answer the following: |  |
| a) | Explain with neat sketch difference between bending moment curve for a ship poised on a wave profile with crest at mid-ship and trough at ends and vise versa. | (8) |
| b) | Explain how a wave profile is obtained for bending moment calculations. | (4) |