## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-II(2006)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain D’Alemberts Principle. |  |
| b) | Draw a neat sketch of “Double Purchase Crab Winch” and explain Mechanical Advantage, Velocity ratio and Efficiency of it. |  |
| c) | A shoft is rotating at 140 rpm. Calculate its velocity in rad/sec and deg/sec. |  |
| d) | What do you mean by “Reversibility of Machine”. State the meaning of load lost in friction and effort lost in friction giving an example. |  |
| e) | Explain the term “Work”. When is “Work” said to be done? How it is different from “Moment” as the units of both are same? Explain with practical example. |  |
| f) | Find the superelevation of outer rail of meter gauge rail track of 7500m radius at velocity of 70 Km/hr. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Use method of joints to determine forces in all the members of the truss shown in the fig.1. |  |
| b) | As bullet leaves the gun with a velocity of 400m/s at an angle of 35 to the horizontal. Find the velocity after 12 seconds. |  |
| c) | A wheel increases it speed from 30rpm to 80rpm in 12 seconds. The wheel diameter is 3m. Find: - Angular Acceleration - Number of revolutions made by the wheel is 10 seconds - Linear Velocity at the point of rim at the end of 10 seconds. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Two bodies of mass 80kg and 20kg are connected by a thread. A force of 400N is applied to the first body of mass 80kg. Coefficient of friction between sliding surface between body and the plane is 0.3. Determine i) the acceleration of the body & ii) Tension in the thread. |  |
| b) | A man of mass 70kg dives vertically downward into swimming pool from a tower of height 20m. He was found to go down in water by 2m and then start rising. Find the resistance force of water. |  |
| c) | A bullet of mass 200gm, is fired horizontally with a velocity of 30m/s from a gun carriage which together with gun has mass of 100kg. The resistance to sliding of the carriage on the surface it rest is 20N. Find - Velocity with which gun will recoil - Distance in which it come to rest - Time taken to do so. | …..2/- |

-2-

| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A spring of stiffness 25N/mm is to be compressed for its length compressing from 80mm to 60mm. If the free length of the spring is 100mm, find the work done in compressing the spring from 80mm to 60mm. If the spring is further compressed by 10mm, find the additional work. |  |
| b) | A body weighing 40N drops freely from a height of 50m and penetrate into the ground by 1m. Find the average resistance to penetration. |  |
| c) | A water tank of 30x103 litre capacity is to be filled in 45 minutes by a pump. The average height is 15m. If pump efficiency is 60%. Find the rated power of the pump. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is Superelavation for the railway on curved path? Draw neat sketch and derive an expression for the angle of bank for railway negotiating a curved path. |  |
| b) | A body of mass 1kg is to be rotated in a vertical circle of 1m radius by means of a string. If the string can withstand a maximum of 120N, find the maximum rpm with which the body can be rotated without breaking the string. Also calculate minimum rpm for maintaining the body in rotation. |  |
| c) | A lift weighing 1200N when empty, is lifted by means of wire rope. A man of mass 75kg is standing in it. Find the tension in the rope when the acceleration of lift is 6m/sec2 while moving upward. Also, determine the reaction acting on the man when the lift is moving downward with same acceleration. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | In a lifting machine, an effort of 14N lift a load of 500N, and effort of 20N lift a load of 900N. Find the Law of Machine, Maximum efficiency, Maximum Mechanical Advantage. The V.R is 96. |  |
| b) | With a neat sketch, explain simple Screw Jack. Derive Velocity ratio for single threaded screw. |  |
| c) | A double purchase crab winch has following dimensions: Effective diameter of load drum = 20cm Length of the handle = 38cm Number of teeth on pinion = 20 and 30 Number of teeth of spurwheel = 75 and 90 When tested, it was found that effort of 90N can lift a load of 180 N and effort of 13.5N lift a load of 315N. Determine - Velocity Ratio of the machine - Law of Machine - Efficiency of the machine. |  |