## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Industrial Electronics & Control Circuits(4202)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw & explain DIAC characteristics. |  |
| b) | Explain natural commutation of SCR with neat diagram. |  |
| c) | Explain the use of TRIAC as a static switch. |  |
| d) | Draw circuit diagram of IC 555 astable multivibrator. Also draw the input and output waveforms. |  |
| e) | Explain the construction and characteristics of LDR. |  |
| f) | Draw the circuit diagram of long duration timer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram, explain the design procedure for high voltage regulator using IC 723. |  |
| b) | Explain the basic regulator using 78xx series IC’s. |  |
| c) | Explain in detail construction, characteristics and operation of SCR. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat circuit diagram and waveforms, explain SCR phase control by Ramp & Pedestal method. |  |
| b) | With neat diagram, explain the operation of series inverter. |  |
| c) | Explain the operation of 2/3 relay type of AC voltage stabiliser. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction & operation of Lead acid battery. |  |
| b) | With neat block diagram, explain the operation of SMPS. Also state its advantages. |  |
| c) | Explain speed control technique of DC shunt motor using field control method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of IC 555 as monostable multivibrator, with neat diagram & waveforms. |  |
| b) | With neat block diagram, explain the operation of Batch counter system. |  |
| c) | Explain speed control technique of single phase induction motor using SCR. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction, characteristics and operation of phototransistor. |  |
| b) | Explain magnetostrictive method of Ultrasonic generation |  |
| c) | Explain the use of Ultrasonics for distance measurement. |  |

***********