## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Electronics Engg./E/EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-I(4134)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | i) What is a modulus of a counter? ii)State the number of flip flops required to design modulus 64 counter. |  |
| b) | Explain digital and analog signals. |  |
| c) | Compare sequential and combinational digital systems. |  |
| d) | Define the following characteristics of logic families: i) Speed of operation ii) Figure of merit iii) Noise immunity. |  |
| e) | Define: i) Flip flop ii) Registers iii) Counters. |  |
| f) | Convert to Binary: i) (25) 10 ii) 23.6. |  |
| g) | Convert to decimal: i) (110101)2 ii) (11001.011)2. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove De’ Morgans first and second theorem. |  |
| b) | Simplify: i) y = ABC+ABC+ABC+ABC ii) A +AB = A + B |  |
| c) | Simplify the following Boolean function in SOP and POS forms f (A,B,C,D) =  (0,1,2,5,8,9,10) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the logic symbol, logic diagram of RS flip flop and write down its truth table. |  |
| b) | Draw the logic symbol, logic diagram of JK flip flop and write down its truth table. |  |
| c) | Draw the logic symbol, logic diagram of D flip flop and write down its truth table. Explain its working. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain different types of registers with logic symbols. |  |
| b) | Explain the working of 4 bit serial-in serial-out register with the help of suitable diagram. |  |
| c) | Draw the logic diagram and explain the operation of 3 bit binary ripple counter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain working of DTL Nand gate. |  |
| b) | Draw the basic circuit of the ECL Logic family and explain its working. |  |
| c) | Draw the logic diagram and explain the operation of a decade counter. Draw the waveforms. |  |
| Q.No.6. | Write a note on any four: | 4 x 3 = 12 |
| a) | Racing condition |  |
| b) | Fan-in and fan-out |  |
| c) | Ring counter |  |
| d) | AND gate, NAND gate, OR gate |  |
| e) | NOR as universal gate |  |
| f) | Tristate Logic |  |

***********