## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT

Subject: Construction-I(4004)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| i) | Explain in brief a) Live load b) Dead load. |  |
| ii) | What is setting out of building? Explain any one method. |  |
| iii) | Define the terms: - Bearing capacity - Load bearing structure. |  |
| iv) | Enlist the merits of concrete over other materials. |  |
| v) | What are the objects of plastering? |  |
| vi) | List the common sizes of doors and windows. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Define the term foundation. Draw the neat sketch of spread footing for load bearing wall. |  |
| ii) | Enlist and explain the common types of building stones used in construction. |  |
| iii) | With the aid of neat sketch explain the terms in stone masonry. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | State the importance of bonds in brick masonry with neat sketch & explain bonding rules. |  |
| ii) | Explain with the neat sketch the features of Flemish bond. |  |
| iii) | Explain the points to be observed in construction of brick masonry. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | Explain in brief about: - Location of door - Function of doors - Standard door size. |  |
| ii) | Explain the requirements of good stair. |  |
| iii) | Explain the thumb rule for geometric design of stair. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| i) | With the aid of neat sketch explain the queen post truss. |  |
| ii) | What is a mazzanine floor? Explain the location and requirements. |  |
| iii) | Explain in detail, the plastering of external wall with sand faced plaster. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| i) | Granite stone flooring |  |
| ii) | Jack arch floor |  |
| iii) | Roof covering materials |  |
| iv) | Pointing and its necessity |  |
| v) | Location of stair |  |
| vi) | Lintel and Arches |  |

*********