## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(4232)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the stress strain curve for mild steel in tensile test showing salient points. |  |
| b) | What are the different elastic constants? |  |
| c) | What is significance of Shear force and Bending moment diagram? |  |
| d) | What do you understand by Beam of uniform strength? |  |
| e) | Derive polar moment of inertia of a circular section? |  |
| f) | Derive an expression to find out shear stress at any layer in a rectangular beam section. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A short steel tube of external diameter 75mm and internal diameter 50mm is surrounded by a brass tube of same length and having external diameter 100mm and internal diameter 75mm. The tubes are rigidly fixed and an axial load of 20KN is placed on tubes. Find the load carried by each. Es= 2x105N/mm2 Eb=1x105N/mm2. |  |
| b) | A steel rod of length 4m and diameter 20mm is being stayed between two plates at a temperature of 60C. Find out the force exerted by the rod after it has been cooled at 20C (a) if plates do not yield and (b) if total yield at two ends is 1mm. Take Es= 2x106Kg/cm2 s=12x10-6/C. |  |
| c) | A vertical circular bar 20mm diameter 4m long carries a tensile load 40KN. Calculate (i) elongation (ii) decrease in diameter (iii) volumetric strain if Poisson’s ratio=0.3 and E=2x105N/mm2. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A steel bar 20mm in diameter, 20cm long was tested to destruction. During the test, following observations were made: load at elastic point = 6500kg, extension at elastic limit = 0.022cm, yield load = 7000kg, maximum load = 13,000kg, final length = 25cm, final diameter = 14mm. Find (i) modules of elasticity (ii) yield stress (iii) ultimate stress (iv) percentage elongation (v) percentage contraction of area. |  |
| b) b) | A steel bar of uniform circular cross section is of length 1m and diameter 50mm is subjected to a tensile force of 10KN gradually applied to it. Find strain energy stored in the bar. Also find its modules of resilience if elastic limit of material of bar is 160N/mm2. Take E=2x105N/mm2. ### OR A solid steel rod of length 1m and diameter 20mm hangs vertically and has a collar securely attached at lower end. Find the maximum stress induced when the weight of 20kg falls on the collar from a height of 150mm. Take E=2x105N/mm2. Also find the absorbed strain energy, modules of resilience and maximum elongation of bar. | ……2/- |

-2-

| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) a) | Find the moment of inertia of the T-section about XX and YY axis passing through C.G. of section and parallel to the base. Flange=100mmx30mm, Overall depth =150mm, Web thickness=30mm. ### OR Find the moment of inertia about the centroidal XX axis and parallel to the base. Angle section 100mmx80mmx20mm as shown in the fig.1. |  |
| b) | A I-beam of overall depth 40mm and flange width 20mm has both flange and web thickness of 20mm. Calculate maximum intensity of shear stress across the section and sketch the shear stress distribution across the section of the beam, if it carries a shearing force of 300KN at a section. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Cantilever ABCD, 7m long is fixed at A such that AB=BC=2m, and CD=3m. It carries loads of 5KN, 3KN and 2KN at B, C and D respectively in addition to UDL of 1KN/m run between A and B. Draw Shear force and Bending moment diagrams. |  |
| b) | A simply supported beam of 8m carries point loads of 5KN, 2KN at distance 3m and 5m from left end. Draw Shear force & Bending moment diagrams. |  |
| c) | An overhanging beam ABC, length 4m, supported on AB of length 3m. The overhanging portion BC is of length 1m. An UDL of 1KN/m is acting over a length of 2m from left support. Two point loads 2KN and 3KN are acting at a distance of 2m from left support and on free end C respectively. Draw Shear force and Bending moment diagram. Also locate point of contraflexure. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A timber joist is 150mm wide and 240mm deep. It is used as a Cantilever of length 4m. What is the maximum concentrated load that it can carry at its free end in addition to its self weight so that the bending stress is not to exceed 7N/mm2. Take unit weight of timber=6KN/m3. |  |
| b) | A wooden beam 2m long is simply supported at its ends and has a cross section 15 cm wide and 60cm deep. It carries an UDL of 90KN/m over its entire span. Calculate the bending stress at a point 20cm above the bottom of beam and 60cm from left support. |  |
| c) | A circular pipe of external and internal diameter 100mm and 80mm respectively is used as a simply supported beam. The span of beam is 4m. Find the safe concentrated load that the beam can carry at the mid span if permissible stress in beam is 120N/mm2. |  |