## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April,2013 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(4335)

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Statical stability curve and information obtained from it |  |
| b) | Precautions to be taken for inclining experiment |  |
| c) | Bale and grain capacity |  |
| d) | Buoyancy curve |  |
| e) | Pivoting and tipping |  |
| f) | Load line marking |  |
| Q.No.2. | Answer the following: |  |
| a) | A box shaped vessel 64m x 10m x 6m floats in salt water on an even Keel at 5 m draught. A forward compartment 6m long and 10m wide extends from the outer bottom to a height of 3.5m and is full of cargo of permeability 25%. Find the new draughts if the compartment is now bilged. | (8) |
| b) | Explain stowage factor and broken stowage. | (4) |
| Q.No.3. | Answer the following: |  |
| a) | Describe with the sketch launching curves used in stern launching of vessel. | (8) |
| b) | What are the important features of the curves? | (4) |
| Q.No.4. | A uniform box shaped barge 40m x 12m beam is divided into four cargo compartments each 10m long. The barge is loaded with 600 tonnes of iron ore level stowed as follows: No.01 hold –135 tonnes No.02 hold –165 tonnes No.03 hold –165 tonnes No.04 hold -135 tonnes The loaded barge floats in fresh water on an even keel draught of 1.75m. Construct the curve of shearing force and bending moment for this condition and also find the position and value of the maximum bending moment for the still water condition and state its type. | (12) |
| Q.No.5. | Answer the following: |  |
| a) | Explain the procedure to carry out GRT and NRT calculations. | (8) |
| b) | List the exempted spaces in tonnage calculations. | (4) |
| Q.No.6. | Answer the following: |  |
| a) | Explain with neat sketch procedure to carry out inclining experiment. | (8) |
| b) | A ship has Km=6.1m and displacement 3150t. When a mass of 15t already on board is moved horizontally along the deck through a distance of 10m it causes 0.25m deflection in a 8m long plumb line. Calculate ships KG. | (4) |

………2/-

-2-

| Q.No.7. | The midship section of a box-shaped vessel is as shown in fig 01. |  |
| --- | --- | --- |
|  | 1.Find the section modulus of the deck and keel. | (8) |
|  | 2.If the vessel is subjected to sagging bending moment of 55,000 t-m, calculate the stress exerted in deck and bottom and state their type. | (4) |