## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C.Machines(4109)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between: - Single layer winding & double layer winding - Front pitch & back pitch - Pole pitch & commutator pitch. |  |
| b) | State the factors on which back e.m.f. of d.c. motor depends. What is the value of back e.m.f. at the time of starting of motor? |  |
| c) | Differentiate between a cumulative compound motor & differentially compound motor giving 3 points of distinction between them. |  |
| d) | State the types of efficiencies in a d.c. generator and define any one of such efficiency of your choice. |  |
| e) | What do you mean by commutation in d.c. machine? Also state the causes of bad commutation. |  |
| f) | Give two points of distinction between d.c. welding generator & ordinary d.c. generator. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State the various parts of d.c. machine and state the material of which each is made of. Also state the functions of only four of its main parts. |  |
| b) | What do you mean by dummy coils where are they used? What is their function? |  |
| c) | State the expression for e.m.f generated in a d.c. generator. Also state clearly each term used in expression. Draw the connection diagram of the long shunt & short shunt compound generator. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | - State the causes of failure of d.c. shunt generator to build up the voltage. 2. Define i) Critical resistance ii) Critical speed in a d.c. generator. | (4) (2) |
| b) | Draw open circuit characteristic curve for a separately excited d.c. generator & explain it in brief. |  |
| c) | Draw the external characteristic of d.c. series generator & explain it in brief. |  |
| d) | A 440V d.c. compound generator has an armature, series field and shunt field resistance of 0.5Ω, 1.0Ω and 200Ω respectively. Calculate the generated voltage while delivering 40A to an external circuit for long shunt connection. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a neat circuit diagram of a bipolar welding generator & explain its working in brief. |  |
| b) | Draw a connection diagram to show parallel operation of d.c. shunt generators and explain its operation in brief. | …2/- |

-2-

| c) | Two 220V d.c. generators each having linear characteristic operate in parallel. One machine has a terminal voltage of 270V on no load and 220V at load current of 35 amperes. While the other has a voltage of 280V at no load and 220V at 40A. Calculate the output of each machine & bus bar voltage when total load is 60A. |  |
| --- | --- | --- |
| Q.No.5. | Answer any three of the following: | 3 x 4 = 12 |
| a) | With the help of neat diagram explain the principle of operation of d.c. motor. How will you determine its direction of rotation? |  |
| b) | State the methods of speed control of d.c. shunt motor. Explain any one such method of your choice in brief with the help of neat circuit diagram. |  |
| c) | What is the necessity of starter in operation of d.c motor? |  |
| d) | Draw speed v/s torque characteristics of d.c shunt motor, d.c. series motor, cumulative compound motor & differentially compound motor. |  |
| e) | Differentiate between d.c series motor & d.c. shunt motor w.r.t their starting torque, conditions of operation, speed variation & two applications. |  |
| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| a) | State the components of armature reaction. What are their effects on flux distribution in d.c. generators? How can these effects be minimized? Explain in brief the method used. |  |
| b) | A 120 V d.c. shunt motor has an armature resistance of 0.2Ω and field resistance of 60Ω. It runs at 1800 r.p.m. when it is taking a full load current of 40A. Find the speed of motor when it is operating on half load. |  |
| c) | - State the expression for speed in d.c.motor. Hence state the factors on which it depends? - State the method used for speed control of d.c. series motor above its normal speed and below its rated speed separately. | (2) (2) |
| d) | Draw the neat diagram of four point starter used in d.c shunt motor starting & label its parts. |  |
| e) | Explain in brief series parallel control used in operation of d.c. series motor. What is its application? |  |

***********