## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(4115)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the factors on which choice of electrical drive depends. |  |
| b) | List the classification of electrical drives. |  |
| c) | Briefly explain the various components of load torque. |  |
| d) | Write the fundamental torque equation of a drive system and name each of the term in the equation. |  |
| e) | List the methods that can be adopted for speed control of an induction motor. |  |
| f) | List the various classes of motor duty classified as per IS 4722. |  |
| g) | Name the motors used for following applications: i) Refrigerator ii) Lifts iii) Machine tools. |  |
| h) | State one application of each of the following: i) Contactor ii) Proximity switch iii) Overload relay. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the concept of steady state stability of a drive system with suitable characteristics and the condition for steady state stability. |  |
| b) | For the mechanism show below, the motor drives the drum through a reduction gear with tooth ratio of 0.25. The frictional torque at the drum shaft is 15 N-m and at the motor shaft is 10 N-m. The motor speed is 1000 rpm. Calculate the equivalent moment of inertia of the drive referred to the motor shaft and also the total motor torque if the gear and motor each have 85% efficiency. |  |
| c) | With a neat diagram and speed-torque characteristics explain the dynamic or rheostatic breaking of a d.c shunt and series motor. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | With a neat diagram explain d.c. dynamic braking of a 3 induction motor. Also draw it speed-torque characteristics. |  |
| b) | With neat diagram and speed-torque characteristics explain the (4 to 8) pole changing method of speed control for an induction motor. |  |
|  |  | ….2/- |
|  | -2- |  |
| c) | With a neat diagram and speed-torque characteristics explain the static Kramer drive method of speed control of a 3 induction motor. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the need of enclosures for drive motor. List the types of enclosures commonly used. Explain any four of them briefly. |  |
| b) | Half hour rating of a motor is 100 KW. Heating time constant is 80 minutes and maximum efficiency occurs at 70% of full load. Determine the continuous rating of the motor. |  |
| c) | Explain the effect of load inertia and environmental factors on the drive motor rating. |  |

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Select drive motors for the following applications and justify with reasons your selection: i) lathe ii) crane. |  |
| b) | With neat diagrams explain the power circuit and control circuit for direct reversal of a 3  induction motor. |  |
| c) | With neat diagram explain the control circuit for sequential starting of two induction motors with time delay. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Write short note on working principle and applications of following: i) Time delay relay ii) Float switch. |  |
| b) | Briefly explain with diagram, basic principle of the following control circuits for: i) interlocking control ii) automatic sequence control. |  |
| c) | Briefly explain the type of drive motors used for following industries with justification: i) Sugar mills ii) Paper mills. |  |

***********