## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Marine Engineering-III(4342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Attempt any five out of the remaining

questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Stud Link Chain |  |
| b) | Deck Capstan |  |
| c) | Goose Neck Pin |  |
| d) | Steam Condenser |  |
| e) | Closing Arrangement of Airdraft on forced Draft system |  |
| f) | Quick Closing valve |  |
| g) | Passive Stabiliser |  |
| h) | Megger |  |
| Q.No.2. | Sketch and describe Freshwater system on board ship. Can such a system be used for seawater as well and where it is used? | (8 + 4) |
| Q.No.3. | Sketch and describe the Bilge OR Ballast water system used on an Inland ore-carrying Vessel. Why the Bilge system is not directly interconnected to the Ballast system? | (8 + 4) |
| Q.No.4. | Sketch and describe an A.C distribution system containing two Alternators, one Turbo Generator and an emergency Alternator with suitable interlocks. When shore connection is also provided what are the safeties incorporated? | (8 + 4) |
| Q.No.5. | Sketch and describe a pedestal mounted Deck crane, used for handling cargo. What are the advantages of crane over the conventional union purchase rig? Are there any disadvantages? What are the safety features fitted on the Deck crane for the purpose of safe operation? | (8 + 4) |
| Q.No.6. | Sketch and describe working of an Anchor Windlass (Manual or Powered) used on an Inland Vessel. What is a chain/cable stopper and why it is required? | (8 + 4) |
| Q.No.7. | Sketch a Domestic Refrigeration system. What is the purpose of a Thermostatic Expansion valve in the refrigeration system? How three different temperatures are obtained for vegetable room, Meat room and Fish room on a Domestic Refrigeration system fitted on board? | (6 + 6) |
| Q.No.8. | Why does control required on the Exhaust Gases emitted by the Main Engine on board ship and how it is achieved? Is there any statutory certificate issued for Emission and how often it needs to be updated/revalidated? | (8 + 4) |

***********