## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Common

Subject: Elements of Electrical Engineering(3001)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the advantages of underground supply system over the overhead supply system. |  |
| b) | State any 5 IE rules relevant to domestic wiring system. |  |
| c) | State the need and purpose of earthing. |  |
| d) | Briefly explain the concept of voltage regulation in a transformer. |  |
| e) | Briefly explain the method of reversal of direction of rotation of a DC shunt motor. |  |
| f) | State the specifications necessary for procurement of 3 induction motor. |  |
| g) | Briefly explain the working principle of a time delay relay. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat diagram explain the construction of a general power cable and functions of each of its layer. |  |
| b) | With a neat diagram explain casing caping type of wiring system and list its advantages & disadvantages. |  |
| c) | An electrical installation consists of 14 light points of 60 Watts each, 5 light points of 100W each, 4 fans of 60W each and one ½ HP single phase pump motor. Assuming that 50% of light and fan points are used for 4 hours/day and the pump motor is operated for 2 hours/day, calculate monthly electricity bill based on a tariff of Rs.120/unit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain the plate earthing system. |  |
| b) | Define a transformer. Explain its working principle and state the characteristics of an ideal transformer. |  |
| c) | Draw a neat labelled diagram of a transformer showing its parts and explain briefly functions of each part. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram briefly explain the working principle of a DC motor and functions of its various parts. |  |
| b) | Draw and explain briefly the working characteristics of a DC shunt motor. |  |
| c) | With a neat diagram briefly explain the working principle of a 3 induction motor and the production of rotating flux. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | List the various types of starters used for starting a 3 Squirrel cage induction motor. Briefly explain the star-delta starter with circuit diagram. |  |
| b) | 1 induction motors are not self starting. Explain the reason. List the methods used for making 1 induction motor self starting. |  |
| c) | With neat diagram explain the working principle & construction of a Universal motor and also mention its applications. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | MCB |  |
| b) | Limit switch |  |
| c) | Switch fuse unit |  |
| d) | Contactor |  |

***********