## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(4234)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Structural Engineers Handbook SP:6(1) & IS 800 is permissible.

5) All dimensions in the figures are in ‘mm’ unless specified.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between a Beam and a Girder. |  |
| b) | Sketch the cross-section of the following steel rolled section (any one). Indicate dimensions clearly. |  |
|  | - ISHB400@ 82.2Kg/m ii) ISBA300@ 47.5Kg/m |  |
| c) | Sketch any six Rivet heads. |  |
| d) | Give the indications of the following in weld representation: i) Site weld ii) Weld all around a part iii) Type of welding process used. |  |
| e) | Calculate the weight of the following steel rolled sections of 1250mm long: - ISA10065 X 8.0mm ii) Mild steel flat 100 x 6.0mm. |  |
| f) | Explain, why unions are not used in flange joint pipelines. |  |
| g) | Differentiate between Vertical stiffeners and Bearing stiffeners in Plate Girders. |  |
| Q.No.2. | Draw the Gusset plates at joints X and Y and determine the exact length of the member X-Y. Refer fig.1. #### NOTE for fig.1 - All members used are ISA5050 x 6.0mm. - Minimum length of 6.0mm Fillet leg used to join each member to the Gusset plate is 200mm. - Gusset plate of 8.0mm thickness are used. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the Isometric view of the portion of the pipeline whose Orthographic views are shown in Fig.2. | (8) |
| b) b) | Explain the importance of piping supports. Sketch a Roller pipe support and state its specialty. ##### OR Explain any two pipe fittings used to incorporate flexibility in a pipeline, with neat sketches. | (4) (4) |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Sketch a Double Riveted (Zig-Zag) Lap joint, joining two plates of thickness 8.0mm and 10.0mm together. Indicate dimensions clearly. |  |
| b) | Sketch a column splicing joining Bottom column ISHB200 @ 40.0Kg/m and Top column ISMB200 @25.4Kg/m. Indicate dimensions clearly. |  |
| c) | Interpret the weld representations shown in fig.3. (any two) |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Draw the orthographic views of the steel structure, whose pictorial view is shown in fig.4. | (8) |
|  | ### NOTE for Fig.4: - All members used are ISA6060 X 6.0mm - All joints are welded Butt joints - No Gusset plates are used at joints. |  |
| b) | Make a bill of materials of the steel structure shown in Fig.4, Q.5a. | (4) |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Different types of clearances used and their importance in Fabrication Drawings |  |
| b) | Difference between strips, sheets, Flats and plates of steel |  |
| c) | Procedure to draw riveted Gusset plate of Truss |  |
| d) | Marking and Naming of members in a structure |  |
| e) | Different types of Lacing systems |  |