## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is soil mechanics? Why knowledge of soil mechanics is important for a Civil Engineer? |  |
| b) | Distinguish between Standard proctor test and Modified proctor test. |  |
| c) | Explain the concept of flow lines and equipotential lines. |  |
| d) | What are the limitations of plate load test? |  |
| e) | Explain how the soil is classified based on values of Cu (Uniformity co-efficient) and Cc (co-efficient of curvature). |  |
| f) | What is Soil exploration? Enlist the methods. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define void ratio, porosity and derive the relation between them. |  |
| b) | A clay fill has a bulk density 1.85gm/cc and moisture content 20% and specific gravity of particles is 2.6. Find the degree of Saturation (SI). |  |
| c) | Explain in detail the I.S. classification of soil. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Darcy’s law and how constant head permeability test is performed in the laboratories. |  |
| b) | A falling head permeameter accommodates a soil sample 8.00cm in height and 50cm2 in cross-sectional area. The permeability of soil expected is 1.1x10-4cm/sec. If it is desired that the head in stand pipe should fall from 50cm to 32cm in 50 minutes, find the size of the stand pipe. |  |
| c) | With a neat sketch explain plasticity chart and A-line diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any two methods of soil compaction. |  |
| b) | In a laboratory compaction test following observations were made: Volume of mould = 1000cc Water content %; 8; 12; 16; 18; 22; 25. Bulk density gm/cc; 1.80; 1.98; 2.20; 2.20; 2.10; 2.16. Plot the water content, dry density curve and calculate optimum moisture content and maximum dry density. |  |
| c) | Explain in detail Local shear failure, General shear failure and punching shear failure. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail triaxial shear test. |  |
| b) | Define folds. How they are classified? Describe any two with sketches. |  |
| c) | Explain plate load test in detail. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Disturbed and undisturbed soil sample |  |
| b) | Cement soil stabilization |  |
| c) | Spring analogy with respect to consolidation |  |
| d) | Failure envelopes |  |
| e) | Properties of Igneous rocks |  |
| f) | Coulomb’s theory of earth pressure |  |

***********