## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-II(4061)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between cylinder blocks of Aluminium and sleeveless Aluminium. |  |
| b) | What are the supercharging limits for S.I & C.I Engines? |  |
| c) | What is the effect of Engine speed on oil consumption? |  |
| d) | List the causes of low oil pressure in I.C. Engines. |  |
| e) | What is sludge? How is it formed? |  |
| f) | An I.C. Engine running at 4200 rpm produces a B.P. of 30 KW. If the loss to cooling water is 2400 KJ/min, calculate the unaccounted losses. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the accelerator circuit of a Solex carburator. |  |
| b) | What is a governor? Describe the Pneumatic governor with a neat sketch. |  |
| c) | What is H.H.V and L.H.V of fuels? Explain an apparatus used to measure the HHV of a liquid fuel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain using neat sketches, the Electronic Ignition System. |  |
| b) | Explain by means of sketches: i)Hot and cold spark plugs ii)Heal Range of spark plugs. |  |
| c) | Describe ‘dwell angle’ and ‘Ballast Resistor’ of an Ignition system and their importance. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketches explain the working of Wankel Engine. |  |
| b) | Explain what you mean by ‘Firing order’ and Balancing of Engine. What is the relation between them? |  |
| c) | With a neat sketch explain the working and advantages of Centrifugal compressor used in Supercharging. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Drawing a neat sketch explain the dual bed catalytic converter. |  |
| b) | Explain any two ways of reducing exhaust emissions by controlling the incoming charge of a SI Engine. |  |
| c) | Explain the Redwood Viscometer with the help of a neat sketch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the ‘Motoring Test’. |  |
| b) | Describe the NDIR method of exhaust gas analysis. |  |
| c) | A 4-cylinder I.C. Engine has 75% mechanical Efficiency. Its BP is 35 KW. Calculate the I.P. and F.P. Assuming that F.P. is constant at all loads, calculate the mechanical efficiency at three quarter of the load. |  |

***********