## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Common

Subject: Elements of Electronics Engineering(3002)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw energy level diagram of P-type and N-type semiconductor. |  |
| b) | Write any three differences between Half wave and full wave rectifier. |  |
| c) | Draw Common Emitter transistor configuration and draw output characteristics. |  |
| d) | What are Barkhausens criteria for sustained oscillations? |  |
| e) | Define any three parameters of OP-AMP. |  |
| f) | Draw symbol of OR gate and write its truth table. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of PN junction diode when it is reverse biased. Also draw reverse biased V-I characteristics. |  |
| b) | Draw circuit of Half wave rectifier and explain working with the help of waveforms. |  |
| c) | Explain the working of Zener diode as a voltage regulator. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw Common Emitter amplifier circuit & explain working with the help of waveforms. |  |
| b) | Explain RC coupling method of cascading of amplifiers. |  |
| c) | Explain with the help of neat diagram 3 bit binary counter. Also draw truth table. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw circuit of Hartley oscillator and explain working with the help of waveforms. |  |
| b) | Draw Astable multivibrator circuit and explain with the help of waveforms. |  |
| c) | Draw block diagram of CRO and explain function of each block. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw circuit of OP-AMP as non-inverting amplifier and explain with the help of waveforms. |  |
| b) | Draw circuit of OP-AMP as subtractor and explain working with the help of waveforms. |  |
| c) | Draw block diagram of signal generator and explain function of each block. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and characteristics of FET. |  |
| b) | Explain the construction and characteristics of DIAC. |  |
| c) | Explain any one application circuit using SCR. |  |

***********