## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Elementary Ship Design(5331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Attempt any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Light ship weight |  |
| b) | Draft to depth ratio of a ship |  |
| c) | Volume carrier and deadweight carrier |  |
| d) | Design features of a container ship |  |
| e) | Specification and contracts |  |
| f) | Cubic number, square number |  |
| Q.No.2. | Estimate the dimensions of a dry cargo vessel of 14000 tonnes DWT at a maximum draft of 8m with a service speed of 15 knots. Given Cd = 0.67 B = L/9 +6 CB = 1.00 – 0.23 VL Where L = Length in meters V = Speed in knots Cd = Deadweight co-efficient CB = Block Co-efficient | (12) |
| Q.No.3. | Estimate the lightship weight of a vessel of service speed of 15.5. knots of dimension 156m x 23.5m x 13.2m. Given the following - Hull weight of a basis vessel of dimension 147m x 24.0m x 12.7m is 6500 tonnes. - Wood and outlet weight of the basis vessel is 300 tonnes. - The block co-efficient of the basis vessel and the proposed design is same at 0.82. - Speed of the basis vessel is 15 knots with engine power of 10,000 BHP. v) Machinery weight = Engine BHP/10 + 200 tonnes. | (12) |
| Q.No.4. | Answer the following: |  |
| a) | Explain how the section area curve of a basis ship is modified for a new position of LCG of a vessel and for a new prismatic co-efficient. | (12) |
|  | ### OR |  |
| b) | Explain what is meant by cargo capacity. What is its significance? Explain how the cargo capacity of a merchant vessel can be estimated at the preliminary design stage. | (12) |

………2/-

-2-

| Q.No.5. | Answer the following: |  |
| --- | --- | --- |
| a) | Explain the specific design features of the following vessel types: - Ore carriers - Passenger vessels. | (4) |
| b) | Explain a typical technical specification of a merchant vessel prepared for the construction of a vessel. | (4) |
| c) | Explain a typical ship design spiral. | (4) |
| Q.No.6. | Answer the following: |  |
| a) | What is meant by a section area curve? Explain how the lines of a vessel are obtained given the section area curve of a vessel along with the associated load waterline curve? | (12) |
|  | ### OR |  |
| b) | What is meant by Gross tonnage and Net tonnage? How is it calculated? Explain the influence of Gross tonnage and Net tonnage on the design of a vessel. | (12) |
| Q.No.7. | Draw a typical G.A. plan of a merchant vessel. Indicate the important machinery, spaces and fittings on board the vessel. | (12) |

***********