## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Architectural Engg.

Subject: Building Construction-III(4524)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any three materials for making of door frames and shutters. |  |
| b) | Draw a simple sketch of RCC eccentric footing & foundation. |  |
| c) | Mention any three areas where timber could be used as a floor material. |  |
| d) | Name any three types of steel trusses. |  |
| e) | Sketch a folded plate roof in RCC with appropriate supports. |  |
| Q.No.2.(A) | Answer any one of the following Questions: | (6) |
| a) | Draw labelled sketches for various types of combined footings. |  |
| b) | Explain the conditions under which eccentric footings are adopted. Support your answer with appropriate sketches. |  |
| (B) | Answer any one of the following Questions: | (6) |
| a) | Explain in detail, procedure for laying & fixing of ceramic tile over existing terrazzo tile in a living room. |  |
| b) | Suggest types of flooring materials to be used for public toilet & assign reasons for your choice. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a cross section through a RCC beam showing reinforcement details. |  |
| b) | Mention the loads taken into consideration while designing the foundation for a structure. |  |
| c) | State the advantages of parliamentary hinge. |  |
| d) | In heavy rain affected areas which floor material you would suggest for garden pavement & why? |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | Draw neat labelled sketches of the following: - Ledged, Braced & Battened door - Louvered window. |  |
| b) | Draw a neat labelled figure of door with steel frame and wooden panel. Draw details of joinery adapted. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention various types of roofing materials and write under which climatic conditions they are adapted. |  |
| b) | Classify trusses according to material & write comparative notes on same. |  |
| c) | Draw & state the role of the following: - Common rafter ii) Jack rafter iii) Ridge tile. |  |

………2/-

-2-

| Q.No.6.A) | Answer any one of the following Questions: | (6) |
| --- | --- | --- |
| a) | Classify roofs according to their shapes and write on suitability of each for various types of buildings. |  |
| b) | Explain the following terms: - Gable end ii) Hipped end iii) Valley. |  |
| B) | Answer the following Questions: | 2 x 3 = 6 |
| a) | Sketch a barrel vault roof supported on RCC beam. |  |
| b) | Sketch three types of pitched roofs in plan. |  |

***********