## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg.

Subject: Energy Management(5108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by harmonics? List any three problems caused by harmonic current in electrical system. |  |
| b) | State benefits of energy conservation to the nation and to the globe. (3 benefits each) |  |
| c) | Briefly explain significance of power analyser in energy audit. |  |
| d) | Define i) lumen ii) lux. |  |
| e) | List any three ways by which the efficiency of transformer can be improved. |  |
| f) | Briefly explain how time based control helps in improving efficiency of lighting system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is load curve? With a neat diagram explain importance of flattening of load curve. Explain any one method. |  |
| b) | State the strategies that can be used to meet future challenges to energy security. Explain the need for such strategies. |  |
| c) | What is energy audit? Briefly explain the need and methodology of preliminary audit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe and state advantages and disadvantages of: i) Block rate tariff ii) Three part tariff. |  |
| b) | The monthly reading of a consumer meter are as follows: Maximum demand = 50KW Energy consumed= 36,000 KWh Reactive energy = 23,400 KVAR If tariff is Rs.200 per KW of maximum demand plus Rs.2 per unit plus Rs.10 per unit for each 1% of p.f. below 0.86, calculate the monthly bill of the consumer. |  |
| c) | Write a short note on energy efficient lighting systems. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain step by step approach for maximum demand control. |  |
| b) | Briefly explain how p.f. can be improved cussing vector diagrams. What are its benefits? |  |
| c) | The load on an installation is 800KW, 0.8 lagging p.f. which works for 3000 hours per annum. The tariff is Rs.300 per KVA plus Rs.2.50/- per KWh. If the p.f. is improved to 0.9 lagging using loss tree capacitor costing Rs.300 per KVAR, calculate annual saving effected. Allow 10% per annum for interest and depreciation on capacitors. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain various methods for determining motor loading. |  |
| b) | Explain procedure for determination of pump efficiency. |  |
| c) | Explain the terms: i) lamp circuit efficiency ii) installed power density iii) average maintained luminance. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of automatic power factor controller. |  |
| b) | Explain soft starter. State its advantages over conventional starter. |  |
| c) | Compare electronic ballast with electromagnetic ballast. |  |

***********