## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(4297)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Name the noxious gases that enter into mine air. State their permissible limits. |  |
| b) | What is the objective of mine ventilation? |  |
| c) | What is meant by Splitting? |  |
| d) | What are the advantages of parallel system of fans? |  |
| e) | Write a short note on: Detection of CH4. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Under what circumstances is CO found in underground coal mines? Write short notes on “CO poisoning”. |  |
| b) | What are the various principles behind the detection of firedamp and its percentage in a mine? |  |
| c) | Write short notes on: - Hopcelite detector - Methane layering - CO2 gas |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mean air temperature in a D.C. shaft 400m deep is 28C and U.C. shaft is 38. Calculate (i) Motive column (ii) The NVP. Assuming average barometric pressure in D.C. shaft to be 750mm of Hg. |  |
| b) | Explain the procedure of conduction pressure survey by aneroid Barometer. |  |
| c) | What are the dangers associated with reversal of main mechanical ventilator? Write short note on “Air crossing”. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Calculate the pressure required in Kilopascal to calculate 2400m3/min of air through a 600m long drift 4 x 2.5m in cross section. Assume K=0.0098NS2m-4 - Write short note on Relative humidity. |  |
| b) | Write short note on: - Angle of Yaw - Atkinson formula on pressure - Laminar and Turbulent flow |  |
| c) | With a neat sketch explain the purpose of Inclined manometer. What is the purpose of splitting? |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Compare auxiliary fan with Booster fan. |  |
| b) | How will you conduct a quantity survey in underground mines? Describe the procedure. |  |
| c) | Describe: i) Parallel ventilation system ii) Series ventilation system. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What is the purpose of installing ventilation doors in a mine? |  |
| b) | Describe briefly about anemometer. |  |
| c) | What are the different methods of coursing of air to a running face? |  |
| d) | 350m3/min of air is entering a district but in the last ventilation connection air is found to be 300m3/min. Calculate VBQ of the district. |  |

***********