## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg./FT/E

Subject: Refrigeration & Airconditioning(5034)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Clausius and Kelvin Planks statement of 2nd Law of Thermodynamics. |  |
| b) | With a neat sketch explain working of Lithium Bromide Absorption system. |  |
| c) | Represent a vapour compression cycle on P-h diagram for following cases: - Vapour is dry and saturated at end of compression - Vapour is wet after compression. |  |
| d) | Sketch the various refrigeration tools. |  |
| e) | Differentiate between high side float valve and low side float valve. |  |
| f) | Explain green house effect. |  |
| g) | Explain Automatic Humidity control in Air conditioning controls. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of cascade system of refrigeration with neat diagram. |  |
| b) | With neat sketch explain the working of vapour absorption system. |  |
| c) | Giving a neat sketch explain the parts of simple vapour compression system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat sketch explain the working of evaporative type of condenser. |  |
| b) | With neat sketch explain any one of a cooling tower. |  |
| c) | Differentiate between Hermetically sealed compressor and open type of compressor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is i) cooling and humidification ii) cooling and dehumidification? Show the process on Psychrometric chart. |  |
| b) | What is a Psychrometric chart? Show the graphical representations of all the properties on the chart separately. |  |
| c) | The atmospheric air has 35C DBT and 25C WBT. Using Psychrometric chart, calculate i) D.P.T. ii) RH iii) SH. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat sketch explain working a water cooler. |  |
| b) | With a neat sketch explain construction and working of cold storage. |  |
| c) | With a neat sketch explain an air handing unit. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the following properties i) Sensible heat ii) DBT iii) WBT iv) DPT v) Specific humidity vi) Relative Humidity. |  |
| b) | What is function of fans and air washers in Air conditioning components? |  |
| c) | Explain dust design method. |  |

***********