## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg./A

Subject: Theory of Machines(4039)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Degree of Freedom. With rigid body in space with no constraints, how many degrees of freedom it will have? |  |
| b) | Define the terms with respect to flywheel: - Coefficient of fluctuation of energy - Coefficient of fluctuation of speed. |  |
| c) | State different factors on which selection of Belt drive depends. |  |
| d) | What is the function of a Governor? In Governors, what do you mean by isochronism? |  |
| e) | Write a short note on Gyroscope and Gyroscopic couple. |  |
| f) | How are natural vibrations different from damped vibrations? |  |
| g) | What are self-energising and self locking brakes? |  |
| h) | What is the function of Gear drive? Compare spur Gear drive and Helical Gear drive. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | In a slider crank mechanism, the crank is 480mm long and rotates at 20 rad/sec in the anticlockwise direction. The length of connecting rod is 1600mm. When the Crank turns 60 from the Inner Dead Centre (I.D.C), determine: - Velocity of the slider - Velocity of point E located at distance 450mm on the connecting rode extended - Angular velocity of the connecting rod. | (6) |
|  | ### OR |  |
| a) | In a Four Bar chain ABCD, the dimensions of the links are as follows: AB=50mm, BC=66mm, CD=56mm and AD=100mm. At a instant when DAB =60, the link AB has a angular velocity of 10.5 rad/sec in the counter clockwise direction. Determine - Velocity of Point C - Velocity of a point E on the link BC when BE = 40mm. | (6) …….2/- |
|  | -2- |  |
| b) | Explain the kliens construction method for finding the velocity and acceleration of piston in Reciprocating engine mechanism. | (6) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A vertical screw with single start square threads 50mm mean diameter and 10mm pitch raised against a load of 5500N by means of hand wheel, the boss of which is threaded to act as a nut. The axial load is taken up by a thrust collar, which supports the wheel boss and has a mean diameter of 65mm. If the coefficient of friction is 0.15 for the screw and 0.18 for the collar and tangential force applied by each hand to the wheel is 140N, find the suitable diameter of the Hand wheel. |  |
| b) | Intensity of pressure on a thrust bearing is not to exceed 40x104 N/m2. The external and internal diameter of the collar is 200mm and 150mm respectively. What should be the minimum number of collars for total axial thrust of 40KN on the bearing? If the shaft rotates at 600 rpm and the coefficient of friction is 0.08, determine the frictional power lost for uniform pressure. |  |
| c) | A single plate clutch transmits 25 KW at 900 rpm. The maximum pressure intensity between the plates is 85 KN/m2. |  |
|  | The outer diameter of the plate is 360mm. Both sides of the plate are effective and coefficient of friction is 0.25. Determine the i) Inner diameter of the plate ii)Axial force to engage the clutch. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | An open Belt drive running at 2.5 m/s transmits 2.5 KW. The angle of lap on the smaller pulley is 165 and coefficient of friction between belt and pulley is 0.3. Determine the effect on power transmission if the initial tension is increased by 10%. | (6) |
|  | ### OR |  |
| a) | A belt drive is required to transmit 18.4 KW from a pulley of 140cm diameter running at 300 rpm. The angle of contact between the belt and pulley is 160 and the coefficient of friction is 0.25. The safe working stress for belt is 315N/cm2. The thickness of belt is 6.5mm and density of belt material is 0.011 Kg/cm3. Determine the belt width required. | (6) |
| b) | With the help of neat sketch explain the construction and working of crank and slotted lever quick return mechanism. | (6) |
|  | ### OR |  |
| b) | With the help of neat sketch explain the working of pickering Governor. | (6) |

………3/-

-3-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Draw the profile of a cam which raises a valve with Simple Harmonic Motion(S.H.M) through 50mm in 1/3rd of revolution. Keep it fully raised through 1/12th revolution and to lower it with SHM in 1/6th revolution. The valve remains closed during the rest of revolution. The diameter of the roller is 20mm and minimum radius of cam is 25mm. The axis of the valve rod passes through the axis of the camshaft. Find the maximum velocity and acceleration of the follower during outstroke when camshaft rotates in clockwise direction with a speed of 240 rpm. | (9) |
| b) | Define train valve. A compound gear train consists of four gears. The number of teeth of gears A, B, C, D are 54, 75, 36 and 81 respectively. Gears B and C constitute a compound gear. Gear A drives gear B and gear C drives gear D. If the gear A rotates at 200 rpm, find the speed of gear D and Train value of the gear train. | (3) |
|  | ### OR |  |
| b) | Define the following gear terminology - Addendum - Module - Pressure angle - Circular pitch. | (3) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Fig 6(a) shows a simple Band-Brake. The brake drum diameter is 220mm and the shaft speed is 240 rpm. The coefficient of friction is 0.3. Find the braking torque when a force of 100N is applied at the lever end. Also determine power lost in friction. |  |
| b) | Explain with a neat sketch working of a prony brake dynamometer. |  |
| c) | Four masses m1, m2, m3 and m4 are 200 Kg, 300Kg, 240Kg and 260Kg respectively. The corresponding radii of rotation are 0.2m, 0.15m, 0.25m and 0.3m respectively and angles between successive masses are 45, 75 and 135. Find the position and magnitude of balancing mass required if its radius of rotation is 0.2m. |  |
|  |  | ……4/- |