## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Power engineering-II(4041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why does 4-stroke cycle engine need heavier flywheel as compared to 2-stroke engine? |  |
| b) | What do you mean by Vacuum Efficiency? Explain. |  |
| c) | The bulb of thermometer measuring wet bulb temperature of air is covered by wet cloth. Why? |  |
| d) | What are the advantages of closed cycle gas turbine over open cycle gas turbine? |  |
| e) | With neat sketches explain different types of Nozzles. |  |
| f) | Compare steam turbines with steam engines. |  |
| g) | Give important applications of gas turbines. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An engine consumes 6 Kg of oil per hour. The calorific value of oil is 30,000 KJ/Kg. If brake power of engine is 22 KW and mechanical efficiency is 85%, find i) indicated power, ii) indicated thermal efficiency and iii) brake thermal efficiency. |  |
| b) | Explain the differences between Spark Ignition Engine and Compression Ignition Engine. |  |
| c) | With neat sketches explain working of a simple carburettor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For engine working on otto cycle, pressure at the beginning is 1 bar and at the end of compression is 10 bar. Calculate compression ratio and air standard cycle efficiency. Assume specific heat ratio =1.4. |  |
| b) | Explain Magneto Ignition System of petrol engine. |  |
| c) | Write a note on Morse test. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of P-V diagram explain velocity compounded impulse steam turbine. |  |
| b) | List down the advantages and disadvantages of reheating and regeneration processes in case of steam turbine. |  |
| c) | Differentiate between impulse turbine and reaction turbine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is critical pressure ratio in case of steam nozzle? Explain its significance giving graph. |  |
| b) | What is a steam condenser? Give its functions and advantages in a steam power plant. |  |
| c) | What are the functions of cooling towers? Give the various types of cooling towers. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a diagram explain the working of a closed cycle gas turbine. |  |
| b) | Explain the working of a Vapour compression refrigeration system with a diagram. |  |
| c) | Draw a neat sketch of a psychrometric chart. Write its salient features. |  |

***********