## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(4331)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is Compulsory. Answer any five from the remaining                            questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Container ship |  |
| b) | Length between perpendiculars |  |
| c) | Effect of shifting of weight on ship’s centre of gravity |  |
| d) | Sheer profile |  |
| e) | Simpson’s first rule |  |
| f) | GZ curve |  |
| g) | LCF |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A ship of 10,000 tonne displacement has its centre of gravity 3m above keel. Masses of 2000, 300 and 50 tonnes are removed from position 1.5m, 4.5m and 6m above the keel. Find the new displacement and position of centre of gravity. |  |
| b) | What is lines plan? Explain with neat sketch. |  |
| Q.No.3. | A ship is floating on even keel at 6.0m draft forward and aft. The areas of water planes are as follows. Draft (m); 0; 1; 2; 3; 4; 5; 6. Area (Sq.m); 5000; 5600; 6020; 6025; 6025; 6020; 6000. Find the ships displacement and vertical centre of bouyancy at this draft. | (12) |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | What are the features of a statical stability curve? Sketch and explain. |  |
| b) | Draw typical cross curves of stability. What information can be obtained from cross curves? Explain. |  |
| Q.No.5. | A box shaped vessel 40m x 6m x 3m is floating in saltwater on an even keel of 2m draft forward and aft. Find the new drafts if a weight of 35 tonnes already on board is discharged from a position 6m from forward. MCT 1cm = 8.4 tonnes metre | (12) |
| Q.No.6. | Given the KN values of a ship at displacement of 40000 tonnes, draw the curves of statical stability, if the KG is at 9.8m. Heel angle; 5; 10; 15; 20; 30; 45; 60; 75; 90. KN(m); 0.9; 1.92; 3.11; 4.25; 6.30; 8.44; 9.39; 9.29; 8.50. Also find (i) Range of stability (ii) Angle of vanishing stability (iii) Maximum GZ (iv) Initial metacentric height. | (12) |

…….2/-

-2-

| Q.No.7. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Construct a TPC curve form the following data. Mean draft (m); 1; 2; 3; 4; 5. TPC (tonnes); 3.10; 4.3; 5.05; 5.5; 5.73. If this ship floats at 2.2m mean draft and then discharges 45 tonnes of ballast, find new draft. |  |
| b) | A ship of 8000 tonnes displacement has KM = 8.0m, KG = 7.5m and has a double bottom tank 15m x 10m x 2m which is full of salt water ballast. Find the new GM if this tank is pumped out till half empty. |  |

***********