## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Candle power ii) Luminous flux. |  |
| b) | Describe the principle of arc welding. |  |
| c) | Explain straight polarity and reverse polarity in DC Welding. |  |
| d) | State the various reasons for failure of heating element in Resistance heating. |  |
| e) | State the characteristics of an ideal traction system. |  |
| f) | Explain infrared heating. Give its application. |  |
| g) | What are the main advantages of Electric heating over other systems of heating? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different methods of heat control of resistance furnace. |  |
| b) | With a neat diagram explain dielectric heating and list its application. |  |
| c) | What is indirect resistance heating? Explain construction and working of any one furnace using indirect resistance heating. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare between DC welding and AC Welding. |  |
| b) | Explain construction and working of any one Arc furnace. State its application. |  |
| c) | Explain phenomenon of Arc blow and list down methods for reducing it. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of speed time characteristics explain the factors affecting the scheduled speed of a train. |  |
| b) | With a neat diagram explain i) The bow collector ii) Pentograph collector. |  |
| c) | Compare the different systems of track electrification. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain importance of following in Lighting design i) Absorption factor ii) Maintenance factor. |  |
| b) | Explain the construction and working of high pressure mercury vapour lamp. |  |
| c) | Define the terms MHCP, MSCP and MHSCP. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on various modes of heat transfer. |  |
| b) | Compare Resistance Welding and Arc Welding. |  |
| c) | Discuss the following with respect to Lamp fittings: i) Dispersive type Reflectors ii) Mirror type reflectors iii) Diffusers. |  |
| d) | Briefly describe the steps involved in designing a heating element. (Derive appropriate expressions) |  |

***********