## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-II(4023)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | State the assumptions made in the theory of long columns by Euler’s method. Also state the expressions to find Euler’s Buckling load and limitations of Euler’s method. |  |
| b) | Using Moment Area method derive the formulas for Fixed End Moments carrying UDL throughout. |  |
| c) | State and prove Distribution Theorem in Moment Distribution Analysis. |  |
| d) | Determine maximum slope and deflection for cantilever beam of span 3m and with a point load of 12KN at free end. Take EI= 10x1012 N-mm2. |  |
| e) | A three hinged arch of 8m span carry UDL of 40KN/m throughout. Find reactions at supports. Rise of arch is 2m. |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | A cantilever beam of 5m span has point loads of 20KN, 30KN and 60KN at a distance of 1m, 2m and 3m from free end of beam. Determine slope at support and deflection under each load and maximum deflection. Take E = 0.14 x 105 N/mm2 , Ixx =19640 x 104 mm4. |  |
| b) | A simply supported beam of 8m span carry a UDL of 50KN/m for 4m length symmetrically placed in middle portion of beam. Determine slop at left support and maximum deflection in beam. Take EI= 22 x 1012 N-mm2. |  |
| Q.No.3. | Answer any one of the following Questions: | (12) |
| a) | A fixed beam of span 8m carry point loads of 25KN, 45KN and 75KN at distances of 2m, 4m and 6m from left support. Find Fixed End Moments and draw SFD and BMD of beam. |  |
| b) | A fixed beam of 5m span carries UDL of 60KN/m throughout the span and a point load of 40KN at 3m from left. Determine Fixed End Moments and draw SFD and BMD. |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | A continuous beam ABC has fixed supports at A and C with AB= 6m and BC=4m. EI is uniform throughout. A UDL of 35KN/m acts throughout the span lengths of AB and BC. Find support moments at A, B and C and sketch SFD and BMD. Use moment distribution method. |  |
| b) | A beam ABC has AB= 6m and BC = 2m with fixed support at A and a simple support at B with BC as overhang. A UDL of 20KN/m acts on BC and point load of 60KN in the middle point of AB. Analyse the beam and sketch SFD and BMD. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Find safe load carried by mild steel rod of 60mm diameter, length 80mm with both ends hinged by Euler’s formula. Take E = 190x103 N/mm2 and factor of safety as 2.5. |  |
| b) | A rectangular strut of 50x80mm in section and length 1.2m with one end fixed and other hinged. Find critical load by Rankine’s formula. Take  = 1/7200 and crushing stress = 300N/mm2. |  |
| c) | Find the shortest length ‘l’ for a pin ended steel column having a cross section 60mm x 150mm for which Euler’s formula applies. E= 2x105 N/mm2 and critical limit for stress = 320N/mm2. |  |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | A three hinged arch of 20m span with both supports at same level has a rise of 3m. An UDL of 70KN/m acts on the right half span and a point load of 100KN at 5m from left end. Find i) Reaction at supports ii) Bending moment at 6m from left. |  |
| b) | A three hinged arch of 30m span with central rise 4m. An UDL of 80KN/m acts throughout the span. Determine i) Reaction at supports ii) Normal thrust, Radial shear at 8m from left end. |  |

***********