## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT

Subject: Irrigation Engineering(4017)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain importance of irrigation. |  |
| b) | What is a weir? What are the situations favouring its construction? |  |
| c) | Differentiate between earthern dam and gravity dam. |  |
| d) | Draw a neat labelled cross-section of Diaphragm type earthern dam. |  |
| e) | Differentiate between base period and crop period. |  |
| f) | Mention and explain in brief any six preliminary and final investigations of a dam site before the project. |  |
| g) | Draw a neat sketch of float type automatic rain gauge. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Mention the three methods of computation of average rainfall in a catchment area and write formula for each with sketches. |  |
| b) | Define Runoff. Explain any three factors affecting runoff. Write two formulae for computation of runoff. |  |
| c) | Write down the factors affecting duty and methods to improve duty. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A channel is to be designed for irrigating 5000 hectares in kharif crop and 4000 hectares in Rabi crop. The water requirement for kharif and Rabi are 60cm and 25cm respectively. The kor period for kharif is 3 weeks and for Rabi is 4 weeks. Determine the discharge of the channel for which it is to be designed. |  |
| b) | What are the functions of a reservoir and mention the factors to be considered for the selection of the reservoir site. |  |
| c) | Explain the causes of water losses from reservoir and suggest measures to reduce water losses. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by gravity dam? Draw a neat sketch of gravity dam and show the various forces acting on it. |  |
| b) | Explain how a gravity dam fails and remedial measures needed for strengthening the same. |  |
| c) | Discuss the practical criteria for design of earth dams. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are spillways? Enlist any three types of spillways. Explain with neat sketch an ogee spillway. |  |
| b) | With a neat sketch explain advantages and disadvantages of bandhara irrigation. |  |
| c) | Sketch and explain in brief about alignment of canal and typical cross-section adopted for carrying water by gravity. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Canal lining – necessity and types |  |
| b) | Open and tube wells |  |
| c) | Barrage – Components and their functions |  |
| d) | Maintenance of canal |  |
| e) | Cross – Drainage works |  |

***********