## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT

Subject: Design of Structures-II(RCC)(4016)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 456-2000 code is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Distinguish between ‘Working stress’ and ‘Limit state’ method of design. |  |
| b) | What do you understand by partial safety factor? State the values with respect to materials and loads as per Indian standards. |  |
| c) | Sketch the reinforcement details as the junction where (i) Column offsets (ii) Beam with break in direction. |  |
| d) | In case of two-way slab, explain the IS specifications for provision of Torsional reinforcement with the aid of sketches. |  |
| e) | Derive the maximum depth of N.A. for balance section with Fe250, Fe415 and Fe500 are 0.53d, 0.48d and 0.46d respectively. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Stating the type of section, determine the moment of resistance for the following data. - Beam section 230mm x 450mm (overall) - Reinforcement 4#16mm - Concrete M25 Grade - Steel Fe415. |  |
| b) | Determine the safe udl under working condition the beam can support in addition to its self weight. The beam is simply supported over an effective span of 5m and reinforced with 4#12mm and beam section is 230mm x 500mm (overall). M20 concrete and Fe415 steel are used. |  |
| c) | Design a rectangular beam to carry an udl of 35KN/m under working condition, in addition to its self weight. The beam is simply supported over an effective span of 4.5m and M25 concrete and Fe415 steel are used. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Describe the mechanism of shear resistance in R.C. beam. - Explain the provision of shear reinforcement in RC beam using ‘Truss and analogy’. |  |
| b) | A simply supported beam of rectangular section 250mm x 460mm (effective). The beam carries a factored udl of 55 KN/m over an effective span of 5m. The beam is reinforced with 5#16mm, M25 and Fe500 are concrete and steel used. Design the shear reinforcement when 2#16 bars are bent-up near the support. |  |
| c) | Calculate the development length Ld for the following in terms of diameter of steel reinforcement in tension. fy (N/mm2); M25; M30; M35. 415. 500. | …….2/- |

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Stating the situation, where do you recommend the doubly reinforced sections. Explain how do you determine the moment of resistance of the section. |  |
| b) | A beam is simply supported over an effective span of 5.5m. The section of beam is 230mm x 400mm (overall) consists of 2#16mm as compression reinforcement and 4#16 mm as tensile reinforcement. If M25 concrete and Fe415 steel are used, determine the safe central point load the beam can support under working condition. |  |
| c) | Design the doubly reinforced section to carry an udl of 45KN/m under working condition over an effective span of 6m. The size of beam is 230mm x 400mm. Take M25 concrete and Fe415 steel. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | A design a continuous one-way slab having thee equal spans of 3.3m each provided for a hall. The live load on slab is 2.5KN/m2 and finishing load is 1KN/m2. Use M20 concrete & Fe415 steel. | (7) |
|  | ### OR |  |
| a) | Design an RC slab for a room 3.5m x 4.5m having two adjacent edges discontinuous and other two edges are continuous. The LL on slab is 2 KN/m2 and finishing load is 1KN/m2. M20 concrete and Fe415 steel are used for the slab. | (7) |
| b) | A corridor 2.6m wide and 9m long is to be provided with R.C. slab. 230mm wide beam supports the slab alround. Take LL on slab as 2.5KN/m2 and finishing load 1KN/m2. Design slab with M25 concrete and Fe 500 steel. | (5) |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | Design a short axially loaded column to carry an ultimate load of 650KN. Effective length of column is 3m. Adopt (i) Rectangular section (ii) Circular section. Use M30 concrete and Fe500 steel. |  |
| b) | Design a square footing to carry an axial load of 500KN from a column 250mm x 250mm. The bearing capacity of soil supporting footing is 180 KN/m2. Take M20 concrete and Fe415 steel. Apply necessary checks for the footing designed. |  |

***********