## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Applied Electronics(4213)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by hie and hfe with reference to a transistor? |  |
| b) | Draw the circuit diagram of transformer coupled amplifier and state its advantages. |  |
| c) | Write a short note on Class A Power amplifier. |  |
| d) | Compare the Hartley oscillator with RC phase shift oscillator. |  |
| e) | What is meant by SSB? List its advantages. |  |
| f) | Define the term CMRR. Write an expression for the same. |  |
| g) | Draw the circuit diagram and frequency response of first order high pass Butterworth filter with Opamp. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw the hybrid equivalent circuit of CE amplifier and briefly explain the same. |  |
| b) | Compare RC coupled amplifier with direct coupled amplifier with necessary circuits and frequency responses. |  |
| c) | Write a short note on Class B push pull power amplifier. |  |
| d) | Explain the demodulation of AM wave using simple diode detector circuit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i)The voltage gain of an amplifier without feedback is 3000. Calculate the voltage gain of amplifier if negative voltage feedback is introduced in the circuit given that feedback fraction is 0.01. |  |
|  | ii)The overall gain of a multistage amplifier is found to be 160. When negative feedback is introduced in the circuit, the voltage gain reduces to 20. Calculate the fraction of output which is feedback to the input. |  |
| b) | Compare the various features of the four types of negative feedback methods using relevant diagrams. |  |
| c) | Explain the working of a Wein Bridge oscillator with the help of circuit diagram and waveform. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of amplitude modulation with waveforms. Draw the frequency response of AM wave and list the various power relations associated with it. |  |
| b) | Describe the demodulation of FM using balanced slope detector method. |  |
| c) | With the help of a neat diagram, explain the various stages in the construction of an Opamp. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | List the characteristics of a practical Opamp. Also explain the virtual ground concept in Opamps. |  |
| b) | Explain the operation of a summing amplifier using Opamp in inverting mode. |  |
| c) | Explain the circuit of Opamp as a differentiator. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Comment on the term ‘slew rate’ in detail. |  |
| b) | Discuss the advantages and disadvantages of direct coupling method used in multistage amplifiers. |  |
| c) | Briefly explain the method to find any one hybrid parameter from the output characteristics of a transistor. |  |
| d) | Explain the laboratory method for determination of CMRR. |  |
| e) | Justify the need for power amplifiers in communication system. |  |

***********