## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Architectural Engineering

Subject: Building Construction-IV(4528)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the functions of a plinth in a building? |  |
| b) | Mention the precautions that are necessary while fixing doors and windows in walls. |  |
| c) | What are the causes of dampness? |  |
| d) | State the functions of a stair. How much minimum space/area is required for a stair in a residential building? |  |
| e) | Sketch a ‘cavity wall’ and write down the functions. |  |
| f) | Write a brief note on “upper floors” with location and function in a building. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the different plinth levels considered for various types of buildings based on soil condition. Explain with suitable examples. |  |
| b) | Give a list of conventional and advanced materials used for door and window openings and mention their merits. |  |
| c) | Draw a neat sketch of a complex door with all components and explain the method of fixing to the wall. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure adopted in preventing dampness from foundation and basement with sketches. |  |
| b) | How waterproofing is done for a ‘wash room’ located in upper floor? Explain in detail. |  |
| c) | Write a brief note on materials used for: i)Damp proofing ii)Waterproofing. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a cross section of a flat roof with a parapet wall and explain the method of water treatment. |  |
| b) | What preventive measures do you suggest to prevent leakage of rain water from a pitched roof? |  |
| c) | State the classification of stairs based on shape and mention their specific uses. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a cross section of a timber stair showing the details of joints between the landing and flight. |  |
| b) | In case of RCC stair, show the details of reinforcement of a flight connecting ground floor and half space landing. |  |
| c) | Explain the following terms used in a stair: i) Stringer ii) Going iii) Riser iv) Baluster v) Navel post vi) Hand rail. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by “Jack Arch Flooring”? Where it is used? Sketch and explain its construction. |  |
| b) | If the timber upper floor has to be constructed, how it is done? Give full details. |  |
| c) | State the merits and demerits of the following upper floors: i) RCC floor ii) Timber floor. |  |

***********