## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C. Machines(4109)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define pole pitch, front pitch and back pitch. |  |
| b) | Explain the constructional features of d.c. welding generator. |  |
| c) | With a neat diagram, explain the working of a simple loop generator. |  |
| d) | What is armature reaction? Mention its two effects. |  |
| e) | Explain the necessary conditions for parallel operation of two shunt generators. |  |
| f) | Explain the importance of back emf of a d.c. motor and give its equation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out the various parts of D.C. machine and their functions and materials used. |  |
| b) | Write a short note on Current Controller of D.C. welding generator. |  |
| c) | Two shunt generators each with an armature resistance of 0.01 ohm and field resistance of 20 ohms run in parallel and supply a total of 4000A. The emfs are respectively 210V and 220V. Calculate the basbar voltage and output of each machine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the classification of D.C. generators. Explain each type in brief with a neat sketch. Also write equation for terminal voltage and armature current for each type. |  |
| b) | The open circuit characteristics of a d.c. shunt generator driven at rated speed is as follows: Field Amperes (A); 0.5; 1.0; 1.5; 2.0; 2.5; 3.0; 3.5. Induced Voltage (V); 60; 120; 138; 145; 149; 151; 152. If the resistance of field circuit is adjusted to 53 ohms, calculate the open circuit voltage and load current when the terminal voltage is 100V. Neglect armature reaction and assume an armature resistance of 0.1 ohm. |  |
| c) | Explain in detail the losses which take place in a d.c. generator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain speed and torque characteristics for series motor. Also give the applications of series motor. |  |
| b) | With a neat diagram explain construction and working of a 4-point starter. |  |
| c) | Explain the different methods of speed control of d.c. shunt motor in brief. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | The armature winding of a 4-pole, 250V d.c. shunt motor is lap connected. There are 120 slots, each slot containing 8 conductors. The flux per pole is 20 mwb and current taken by the motor is 25A. The resistance of armature and field circuit are 0.1 and 125 ohm respectively. If the rotational losses amount to 810W, find (i) Gross torque (ii) Useful torque and (iii) efficiency. |  |
| b) | Define (i) Electrical Efficiency (ii) Mechanical Efficiency and (iii) Overall efficiency with respect to motor. Also explain why d.c. series motor should not be started on No load. |  |
| c) | Compare shunt, series and compound motors w.r.t starting torque, with the help of characteristics. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is commutation in d.c. machine? What are the different methods to improve commutation? |  |
| b) | Explain the effect of armature reaction on the performance of d.c. machine. |  |
| c) | Mention and explain the various causes for the failure of the generator to build up the voltage. |  |

***********