## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe i) Wind energy ii) Energy from Biogas. |  |
| b) | What is calorific value of a fuel? |  |
| c) | State & explain Zeroth’s law of thermodynamics. |  |
| d) | What is the specific heat of the gas? |  |
| e) | What are the advantages of superheating the steam? |  |
| f) | Define i) Boiler Mountings ii) Boiler Accessories iii) Boiler Shell. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The percentage composition of a certain fuel by mass is C=87.1; H=4.4; O=1.2 and ash=7.3. The percentage volumetric composition of dry flue gases is CO2=15; CO=2.2; O=1.6; N2=81.2. Estimate the mass of flue gases produced per Kg of fuel burnt. |  |
| b) | Explain with a neat sketch: - Kelvin – Planck Statement - Clausius Statement. |  |
| c) | State and Explain Boyle’s law and Charle’s Law and prove that the characteristics gas equation is PV=mRT. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the formation of steam and explain how the wet steam, dry saturated & superheated steam is produced. |  |
| b) | Steam at 18 bar and dryness 0.9 is heated at constant pressure until dry and saturated. Find the increase in volume, heat supplied and work done per Kg of steam. |  |
| c) | Steam enters an engine at a pressure of 12 bar with a 67C of superheat. It is exhausted at a pressure of 0.15 bar 0.95 dry. Find the drop in enthalpy of the steam. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch “La Mont Boiler”. |  |
| b) | Describe with a neat sketch the pressure gauge used in a boiler. |  |
| c) | What is boiler draught? Explain different types of draughts. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch construction and working of Air Preheater. |  |
| b) | Explain with a neat sketch Axial flow air compressor. |  |
| c) | Determine the entropy per Kg of superheated steam at a pressure of 20 bar and a temperature of 250C. Assume Cp for superheated steam as 2.2 KJ/KgK. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Mollier diagram & its construction |  |
| b) | After cooling |  |
| c) | Air motor and its applications |  |
| d) | Velox-Boiler |  |
| e) | Economiser used in Boilers |  |

***********