## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg.

Subject: Maintenance Engineering(5035)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Indicate with a symbol a surface having surface Roughness 45 m, sampling length 2.5 mm, perpendicular lay direction. |  |
| b) | What is a claw coupling? How do we use it and align it for proper functioning? |  |
| c) | What are the grades of oil specified for use in - Gear Box 2) Engine 3) Chain drive. Give grades of oil in SAE units only. |  |
| d) | What is meant by ‘dynamic balancing’ of repaired parts? |  |
| e) | What is Repair complexity number? How can this be used in preparation of estimates? |  |
| f) | What is a machine signature? Explain. |  |
| g) | Differentiate between long form and short form work order. (3 differences) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | “It is essential to maintain records by persons handling maintenance”- Justify the statement. |  |
| b) | If x = x0 sinwt represents displacement in vibration analysis, with commonly used notations, explain the various terms involved. How will be the velocity and acceleration expressed? |  |
| c) | Write explanatory notes on: - Stroboscope - Velocity pick-up. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Giving one example each to explain how each of the following is arrived at: - Estimating time of maintenance - Estimating Labour cost. |  |
| b) | Draw a representative diagram of EOQ v/s Time and explain the following terms: i) Lead Time ii) Reorder point iii) Economic Order quantity iv) Reserve stock. |  |
| c) | How will you report a problem, say that of pump not working in a plant, as a report writing exercise? Write this report. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how you will carry out: - Recovery of a worn out shaft - Recovery of a worn out sliding bearing. |  |
| b) | What are the requirements of a good Lubricant? |  |
| c) | Where are filters commonly used in industrial machines and how do we clean them? |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain any two methods of adjusting belt tension with neat sketches. |  |
| b) | What are the ideal conditions for noting linear measurements? Explain any two types of joint callipers used for this purpose. |  |
| c) | Briefly describe the working of a stylus probe instrument with a sketch. |  |
| Q.No.6. | Writes short notes on any four: | 4 x 3 = 12 |
| a) | Servicing of Pneumatic valves |  |
| b) | Standardisation of spares in maintenance |  |
| c) | Shut-down maintenance |  |
| d) | Predictive maintenance |  |
| e) | Work simplification for planning of activities |  |
| f) | Modes of Lubrication |  |

***********