## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mining Engg.

Subject: Underground Mining Machinery(4294)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List out different types of ropes used in Mines. |  |
| b) | Sketch & label winding sheave and state the purposes of winding. |  |
| c) | Write suitability conditions for Shearer. |  |
| d) | Write suitability conditions for road header and continuous miner. |  |
| e) | Compare diesel Locomotive with battery Locomotive. |  |
| f) | Tension arrangement in endless rope haulage. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Explain construction of stranded rope. | (6) |
| b) | Explain reliance cape. | (3) |
| c) | Sketch and label king safety hook and state purpose of it. | (3) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain Torque & time diagram for cylindrical drum. |  |
| b) | Compare drum winding with Koepe winding. |  |
| c) | Explain different types of mechanical brakes. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain coal cutting machine. |  |
| b) | Explain roof bolting machine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain endless rope haulage. |  |
| b) | A Locomotive hauls a train of 16 mines car each weighing 1 tonne up an incline of 1 in 25 at a constant speed 13.5 Km/hr. If the locomotive weighs 6 tonnes and rolling friction is 1/60, what must be power of the locomotive? |  |
| c) | What is the least gradient for a self acting incline 300m long to work with 10 tubs on each train, each tub weighing 0.25 tonnes and carrying 0.6 tonnes of coal. The rope weighs 0.75 Kg/metre. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Shaft guide |  |
| b) | Shuttle car |  |
| c) | Superelevation |  |
| d) | Locomotive |  |

***********