## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electronics Engineering/EC/MED/E&EE

Subject: Analog Electronics(4140)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Slew rate ii) Input offset voltage iii) CMRR. |  |
| b) | What are the advantages of active filters over passive filters? |  |
| c) | Define a filter. How are filters classified? |  |
| d) | Draw a neat circuit diagram of an integrator and show the output waveform for input as sine wave, square wave and triangular wave. |  |
| e) | Time scale the following differential equation. d2ydt2+dydt+104y=1000u(t) |  |
| f) | Draw the set up to solve the following equation d2ydt2+9.2dydt+0.35y=c(t), with initial condition y(0)=4.8 and dydt(0)=−2.3. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive the closed loop voltage gain for the voltage series feedback amplifier. |  |
| b) | Given R1 = 1K, RF = 10K for a closed loop non– inverting amplifier, calculate AF, , RIF, ROF, fF. Vsat = 15V, Ri = 2M, Ro = 75, fo = 5Hz, A = 2 x 105. |  |
| c) | With a neat diagram and derivation explain the summing, scaling and averaging amplifiers in inverting configuration. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram and waveform explain the working of RC phase shift oscillator using Op-amps. |  |
| b) | Explain the operation of Op-amp as an astable multivibrator. |  |
| c) | Write a short note on differentiator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of virtual ground in Op-amp. |  |
| b) | With neat diagram and waveform explain Op-amp as a clipper. |  |
| c) | Explain the frequency response of (i) low pass filter (ii) high pass filter (iii) Band pass filter (iv) Band reject filter. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat diagram and waveform explain the operation of Schmitt trigger circuit. |  |
| b) | Explain capture range and lock range with respect to transfer characteristics of PLL. |  |
| c) | Explain the over load protection and thermal shutdown in IC voltage regulators. |  |
| Q.No.6. | ### Write short note on any three: | 3 x 4 = 12 |
| a) | Zero crossing detector |  |
| b) | Voltage controlled oscillator |  |
| c) | Fixed voltage regulators |  |
| d) | Amplitude scaling |  |

***********