## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(4022)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is modular ratio? Write moment of resistance formula for flitched beam. |  |
| b) | What do you mean by statistically indeterminate structure? State their uses and examples. |  |
| c) | Define limit of eccentricity. Give the limit of eccentricity for rectangular section. |  |
| d) | What do you mean by internal and external stability of structure? |  |
| e) | What is the purpose of ILD and rolling loads? |  |
| f) | State the assumptions made in theory of torsion. |  |
| g) | Explain significance of Mohr’s circle in analysis of complex stress system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define flitched beam. Calculate moment of resistance of flitched beam formed by timber joist of 200mm width and 250mm depth, stiffened with two steel plates each of 200mm width and 12mm thick attached at top and bottom of wooden joist symmetrically. Take Es = 16Et and allowable stress in timber and steel are 75 N/mm2 and 6N/mm2 respectively. |  |
| b) | A beam is formed by joining two bars, one of brass and other of steel, each 60mm wide and 12mm deep. The beam is supported over a span of 1.5m, with the brass bar placed over the steel bar. Determine the maximum load, which can be applied at the centre of the beam when the bars are separate and can bend independently. Es = 2 x 105 N/mm2, Et = 0.8 x 105 N/mm2, s = 112.5 N/mm2 and b = 75 N/mm2. |  |
| c) | Determine the degree of indeterminancy of the following structures: - A portal frame with both supports fixed - A propped cantilever - A King Post truss with one support roller and other hinged - A two hinged arch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A short C.I column of hollow circular section has a projecting bracket carrying a load of 80 KN, the load line is off the axis of column by 300mm. The size of column is 400mm outside diameter and 300mm internal diameter. Calculate the maximum and the minimum intensities of stresses in the column section. |  |

………2/-

-2-

| b) | A rectangular pier of 1500mm x 2700 (deep) is subjected to an eccentric load of 900KN. Eccentricity of load below X-X axis is 400mm and from YY axis is 300mm. The load is placed in 1st Quadrant. Calculate the maximum and minimum stress intensities in the section. |  |
| --- | --- | --- |
| c) | At a point in a strained material the intensities of stresses across two planes right angles to each other are 100 N/mm2 on X-axis (tensile) and 60 N/mm2 (tensile) on Y axis. A shear stress of 40 N/mm2 acts across both planes. Evaluate the magnitude and direction of principal stress and maximum shear by analytical method. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a point in a beam, there is a bending stress of 30N/mm2 (tensile) and shear stress of 20 N/mm2 across the planes. Determine the magnitude and direction of principal stresses as well as maximum shear stress. |  |
| b) | The principal stresses at a point in the section of a member are 125 and 50 N/mm2, both tensile. Locate the plane across which the resultant has maximum obliquity. Use Mohr’s circle. |  |
| c) | Find shear force at midpoint of simply supported girder of span 7m carrying load of 90KN, 80KN and 70KN at a distance of 1m, 2m and 3m respectively from left support. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw influence line diagram for following cases: - ILD for beam end reaction of span ‘L’ - ILD for S.F. at any section of span ‘L’. |  |
| b) | A girder of span 18m is simply supported. A wheel load in the force of UDL of length 6m and magnitude 40KN/m is rolling on the girder. Find maximum shear and bending moment in the girder using influence line diagram. |  |
| c) | A girder of span 40m is carrying a rolling Udl of 150KN/m of length 7m. Determine maximum shear force and BM at a section of 12m from left end. Used ILD concept. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid shaft is subjected to a torque of 4500Nm. Find the necessary shaft diameter if shear stress is not to exceed 50N/mm2. Also find the power generated by the shaft if it rotates at a speed of 120 rpm. |  |
| b) | A hollow circular shaft 500mm long, with an internal diameter of 50mm and thickness of metal 30mm, is rotating under a pure torque T. If the maximum shear stress is 70 N/mm2, evaluate T and the angle of twist. G = 80 N/mm2. |  |
| c) | A solid circular steel shaft has to transmit 400 KW at 250 rpm. The maximum shear stress is not to exceed 50 N/mm2 and the angle of twist must not be more than 1 in a length of 3m. Select a suitable diameter. G= 80N/mm2 |  |

***********