## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Chemistry of Welding Processes(4231)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | State any three advantages of liquid oxygen. |  |
| b) | Why is Helium gas better suited as a shielding gas during welding of underside of a job? |  |
| c) | How is acetylene checked for the presence of Hydrogen Sulphide in it? |  |
| d) | What is the effect of presence of air as impurity in acetylene? |  |
| e) | Mention the various fuel gases in use other than Acetylene. |  |
| f) | Write the chemical reaction for the combustion of acetylene. |  |
| g) | Why is efficiency of welding better with rightward technique of welding? |  |
| h) | What are non–corrosive soldering fluxes? |  |
| i) | State the type of fluxes used for welding Aluminium and its alloys. |  |
| j) | State the disadvantages of using bare wire electrodes. |  |
| k) | Write the formula for the calculation of carbon equivalent for alloy Steel. |  |
| l) | Mention any three ways of preventing gaseous contamination. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the Linde’s method of production of oxygen by liquefaction of air. |  |
| b) | Giving chemical equations state any two characteristics of acidic oxides. |  |
| c) | Discuss the significance of process of oxidation in welding. |  |
| d) | Explain the need for preheating of carbon dioxide during welding. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat labelled diagram of cross section of acetylene generator and explain the need for the use of porous fillers in the cylinder. |  |
| b) | Write a note on explosibility of pure Acetylene. |  |
| c) | Discuss the effects of any four impurities in Acetylene. |  |
| d) | Define the terms: |  |
|  | (i) Calorific Value (ii) Ignition temperature |  |
|  | (iii) Flame temperature (iv) Flame velocity w.r.t. a fuel. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat labelled diagram of oxyacetylene flame and write the reactions taking place in inner and outer cone of the flame. |  |
| b) | Discuss the applications of any two types of oxyacetylene flames. |  |
| c) | Explain the basic difference between: |  |
|  | (i)Brazing and soldering |  |
|  | (ii) Welding and Brazing. |  |
| d) | Discuss the functions and requirements of brazing flux. | ………2/- |

-2-

| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Discuss the functions performed by various Electrode coating flux ingredients. |  |
| b) | Explain the difference between consumable and non-consumable welding electrode and write a note on care of welding electrodes. |  |
| c) | State the sources of the gases absorbed by molten weld metal and explain its effects. |  |
| d) | How are electrodes manufactured by Extrusion process? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the mechanism of pore formation due to absorption of gases by welds. |  |
| b) | Discuss the two types of gas metal reactions due to absorption of gases by welds in brief. |  |
| c) | State any four points of differences between Metals and Non-metals. |  |
| d) | Draw a neat diagram of B.C.C. and F.C.C. crystal lattice structure and give their brief description. |  |

***********