## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Common

Subject: Elements of Civil Engineering(3003)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List various types of roof covering used for pitched roofs. |  |
| b) | List various materials used for Acoustic treatment. |  |
| c) | Define i) Bench mark ii) Datum iii) Reduced level. |  |
| d) | Mention thumb rules for depth and width of foundation. |  |
| e) | What is RCC? List ingredients of concrete. |  |
| f) | What is local attraction? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by profile levelling? Explain how will you carry out survey of the same. |  |
| b) | Bearings of sides of closed traverse is as follows. Calculate included angles. Line FB AB N 5610E BC S 5040E CD S 1950W DA N 7040W |  |
| c) | Explain with sketch procedure of indirect ranging. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following consecutive readings were taken with levelling instrument at an interval of 20m. Find RL of all points. 2.375, 1.635, 0.61, 3.635, 2.845, 2.08, 1.830, 0.97, 0.435, 1.645, 2.25, 3.45 & 3.935. The instrument was shifted after 4th & 8th reading. RL of starting point is 60.2m. |  |
| b) | Explain procedure you will follow in measuring horizontal angle using theodolite. |  |
| c) | Name two methods of reduction of levels. Give comparison between them. Also state suitability of each method with example. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain procedure how vertical angles are measured using theodolite. |  |
| b) | Explain with sketch various forms of steel roof truss. Also compare wooden & steel roof truss. |  |
| c) | Write short note on Thermal insulation. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List various types of flooring. Which type of flooring is suitable for factory, workshop, auditorium and office buildings? |  |
| b) | Explain following stages of concreting operation (i) Mixing (ii) Curing. |  |
| c) | Draw a neat cross-section of Raft foundation & grillage foundation. Under which situation each is applicable? |  |

………2/-

-2-

| Q.No.6. | Using line plan shown in Fig.1, draw to suitable scale Plan, elevation, section. Assume necessary data if required. | (12) |
| --- | --- | --- |