## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Architectural Engg.

Subject: Structures-I(4537)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between resolution and composition of forces. |  |
| b) | Explain the principles of transmissibility of forces. |  |
| c) | State the difference between solid and skeletal structure. |  |
| d) | Define the term shear force and bending moment. |  |
| e) | Enlist the causes for crack development in structure. |  |
| f) | Find the value of maximum bending moment incase of cantilever beam of span (l) subjected to load W at the free end. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Outline the steps involved in the analytical method for the determination of resultant of co-planar forces system. |  |
| b) | For a system of concurrent co-planar forces, if the resultant is Zero, find the magnitude and direction of force P. Refer fig.1. |  |
| c) | Explain the different grades of concrete and steel used in Civil Engineering construction. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how the structural element behaves when it is subjected to (i) Compressive force (ii) Tensile force. Explain with example. |  |
| b) | Draw a neat sketch of load bearing wall with foundation and explain the guidelines for approximate design of foundation. |  |
| c) | Locate the centroid of the section, shown in fig.2. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the term “beam” and with the help of neat sketch explain (i) Cantilever beam (ii) A fixed beam (iii) Overhang beam (iv) Sagging and hogging moment. |  |
| b) | Draw the BMD and SFD for a cantilever beam of span 2m subjected to a UDL of intensity 5 KN/m for entire span and a point load of 10 KW acting at 1m from fixed end. |  |
| c) | A simply supported beam of span 8m carries three point loads of 20KN, 30KN & 40KN acting at a distance of 1m, 3m & 6m respectively from left support. Draw the BMD and SFD. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail how these factors contribute to the occurrence of cracks in structure: (i) Elastic deformation (ii) Movement and soil settlement. |  |
| b) | Explain in detail, the importance of good architectural design and choice of material in prevention of cracks in the structure. |  |
| c) | Locate the centroid of the area ABCDE shown in fig.3. |  |

………2/-

-2-

| Q.No.6. | Write a short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Concrete mix design |  |
| b) | Working stress and limit state methods |  |
| c) | Objects of rehabilitation and repair |  |
| d) | Structural and non-structural cracks |  |