## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Mechanical Engg./P

Subject: Tool Engineering-II[Press Tools](5077)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Design and draw the simple blanking die to blank the work piece shown in fig.1, giving - |  |
| --- | --- | --- |
|  | (i) At least two view of assembly | (16) |
|  | (ii) Detailed drawing of punch and die block (two view each) | (8) |
|  | (iii) Press tonnage & stripping force calculations | (6) |
|  | (iv) Bill of material. | (5) |
|  | (Take shear strength of sheet =385 N/mm2) |  |
|  | ### OR |  |
| Q.No.1. | Design and draw the simple piercing die to pierce 16  hole in the pieces shown in fig.1,giving- |  |
|  | (i) At least two view of assembly | (16) |
|  | (ii) Detailed drawing of punch and die block (Two views each) | (8) |
|  | (iii) Die clearance calculations | (6) |
|  | (iv) Bill of material. | (5) |
|  | (Take shear strength of sheet =385 N/mm2) |  |
| Q.No.2. | Answer any three: | 3 x 8 = 24 |
| a) | Give two different strip layouts for blanking the work pieces shown in fig.1. Assume the length of strip to be 1250mm. Also, calculate percentage material utilisation in each case. |  |
| b) | Describe the construction and working of a progressive die giving neat sketch. |  |
| c) | Describe two types of strippers giving neat sketches. |  |
| d) | Describe an inverted die giving neat sketch and state its advantages. |  |
| e) | Explain the effects of following factors on drawing operation: |  |
|  | (i) Blank holder pressure |  |
|  | (ii) Radius on drawing punch & die. |  |
| Q.No.3. | Answer any two: | 2 x 8 = 16 |
| a) | Calculate bending forces for the work pieces shown in fig.2. Ultimate tensile strength for material =448 N/mm2. |  |
| b) | Describe the following types of bending giving neat sketches: |  |
|  | (i) Edge bending (ii) U-bending. |  |
| c) | Describe in detail metal flow during a drawing operation, giving neat sketches. |  |
| d) | What is meant by bulging? Explain the construction & working of Urethane bulging die. |  |