## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT

Subject: Construction-II(4009)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the problems associated with deep excavations? |  |
| b) | What is pre-stressed concrete? |  |
| c) | Mention any five acoustic materials used in building construction. |  |
| d) | Draw a labelled sketch showing pit method of underpinning. |  |
| e) | Draw a neat sketch of back trench hoe excavator. |  |
| f) | Mention various fire protection systems used in multistorey buildings. |  |
| g) | Briefly explain any one method of waterproofing of roof slabs in Goa. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of well point system showing essential parts and explain the process of dewatering of trenches using well point systems. |  |
| b) | What are the precautions to be taken for safety of foundations in black cotton soil? |  |
| c) | With the aid of a neat sketch explain the necessity and construction details of the following: (i) strap footing (ii) plinth beams. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a ‘Raft foundation’ and explain its construction details. |  |
| b) | Differentiate between end bearing piles and friction piles. Draw sketches showing the reactions offered by these piles. |  |
| c) | Explain in detail process of installing a Simplex pile. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any two methods adopted for corrections of tilts and shifts in caisson construction. |  |
| b) | What is shoring? State different types of shoring methods. Explain Raking or inclined shores with aid of neat sketch. |  |
| c) | Discuss briefly the causes and effects of dampness. State different materials used for prevention of dampness. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are cofferdams? What are the different types of cofferdams? Explain use of Dikes as cofferdams. |  |
| b) | Describe various types of partition walls used in construction practice. |  |
| c) | Explain working of any two of the following construction machinery and situations under which it is used. (i) Power Shovel (ii) Bulldozer (iii) Hydraulic jacks. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Types of Concrete Mixers |  |
| b) | Derrick cranes |  |
| c) | Runners for timbering of trenches |  |
| d) | Isolated footing v/s Combined footing |  |
| e) | Pre cast pile |  |
| f) | Shapes of wells |  |

***********