## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For transformer, define - H.V winding - Step up transformer. |  |
| b) | State emf equation of a transformer. Also define its turns ratio. |  |
| c) | The primary winding of a 500KVA, 3300/400 volt transformer has 800 turns. Determine - The number of turns of secondary winding - emf per turn. |  |
| d) | Draw a phasor diagram of transformer for no load operation. |  |
| e) | List any three condition for satisfactory operation of single phase transformer in parallel. |  |
| f) | State two advantages and one disadvantage of Auto-transformer over two winding transformer. |  |
| g) | With a neatly labelled diagram briefly explain Scott connected transformer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain importance of following components in transformer - Breather - Level indicator. |  |
| b) | With a neat diagram explain working of OFF load Tap changer. |  |
| c) | Explain cooling of transformer by ‘ONAN’ type method. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the equivalent circuit of a transformer and explain its importance. Show how the equivalent circuit can be simplified without introducing much error. |  |
| b) | A single-phase transformer has 1200 turns on the primary and 300 turns on secondary. The no load current is 2.5 Amp at 0.2 p.f. lagging. Calculate the current and power factor of primary circuit when secondary draws a current of 300 Amp at a p.f. of 0.8 lagging. Neglect voltage drop in windings. |  |
| c) | A 2000/200 volt, single-phase transformer has primary resistance and reactance of 2.0 ohms and 4.0 ohms respectively. The corresponding secondary values are 0.025 ohms & 0.05 ohms. Determine - total resistance as referred to Primary - total reactance as referred to secondary. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define per unit quantity. Selecting Base KVA and Base KV deduce expression for base impedance and per unit impedance. |  |
| b) | Define voltage regulation of a transformer. Deduce the expression for percentage voltage regulation. | …….2/- |

-2-

| c) | The primary and secondary winding of 30KVA, 6000/230 volt, single phase transformer have resistance of 10 ohm & 0.016 ohm respectively. The total reactance of transformer referred to primary is 23 ohms. Calculate percentage regulation of transformer while supplying full load current at a p.f. of 0.8 lagging. |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are various losses in transformer? Where and how they occur? How do they vary with load? |  |
| b) | A 25 KVA transformer has iron and half full load copper losses of 350 Watt and 100 Watt respectively. Calculate its efficiency at - Full load and upf - Half full load and 0.8 p.f. lag. |  |
| c) | Find all day efficiency of a transformer having maximum efficiency at 98% at 15 KVA and unity power factor and loaded as follows - 12 hours – 2 KW at 0.5 p.f. lagging - 6 hours – 12 KW at 0.8 p.f. lagging iii) 6 hours – No load. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat diagram the procedure of connecting two three phase transformers in parallel. |  |
| b) | Two 110,000/33,000 volt transformers are operated in parallel to share a load of 125 KVA at 0.8 p.f. lagging. Transformer details are Transformer 1: 100 KVA, resistance = 0.9%, reactance=10% Transformer 2: 50 KVA, resistance = 1%, reactance=5% Find load shared by each transformer. |  |
| c) | Write a short note on furnace transformer. |  |

***********