## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A 3, 6 pole induction motor is supplied from a 50Hz, 400V supply. Calculate i) Synchronous speed and ii) the speed of the motor when the slip is 4%. |  |
| b) | Explain in brief the method of starting slip-ring induction motor. |  |
| c) | Explain the classification of 1 induction motor based on methods of starting used. |  |
| d) | What are the advantages of rotating field and stationary armature system in case of alternator? |  |
| e) | Explain the method of synchronisation of Alternators by 3 Lamp method. |  |
| f) | Explain the concept of synchronous condenser. |  |
| g) | Describe in brief the operation of Reluctance motor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the two types of rotor construction of 3 induction motor. |  |
| b) | Derive the expression for torque of a 3 induction motor under running condition and thus derive the condition for maximum torque under running condition. |  |
| c) | The power input to a 3 induction motor is 50KW and the corresponding stator losses are 2KW. Calculate i) the total mechanical power developed and the rotor I2R loss when the slip is 3% ii) the output horse power of the motor if the friction and windage losses are 1KW. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the equivalent circuit diagram of a 3 induction motor. |  |
| b) | Explain the effect of variation of supply voltage, frequency and open delta condition on torque and speed of a 3 induction motor. |  |
| c) | Upto what HP rating induction motor can be started with star-delta starter. Hence explain the star-delta starter. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of resistance split phase induction motor. |  |
| b) | Give the advantages and applications of capacitor start and capacitor start and run, single phase induction motors. |  |
| c) | Explain the construction and working of alternator. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the open circuit and short circuit characteristics of an alternator. Explain the shape of the characteristics. |  |
| b) | Explain the armature reaction at unity and lagging p.f. loads in an alternator. |  |
| c) | A 2000KVa, 6600V, 3 star connected alternator has a resistance of 0.4 ohm per phase and a synchronous reactance of 4.5 ohm per phase. Calculate the percentage change in voltage (terminal) when the rated output of 2000KVa at a p.f. of 0.8 lagging is switched off. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the conditions and advantaged for parallel operation alternators? |  |
| b) | Explain the effect of change of excitation of a synchronous motor on its power factor. |  |
| c) | Explain the construction, advantages and applications of universal motor. |  |

***********