## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures-I[Steel](4024)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 800-1984 code and steel tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | State merits and demerits of steel structures and list the fields of applications. |  |
| b) | State the situations, where plate girders are used and explain its various components with the aid of neat sketch. |  |
| c) | With the aid of stress-strain curve, explain the salient points for mild steel. |  |
| d) | Explain what do you understand by “web crippling”. |  |
| e) | What are the advantages and disadvantages of tubular sections? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a lap joint to connect two plates 100mm x 12mm and 100mm x 8mm by fillet welds. Take tensile stress for plates as 150MPa and shear stress for welds 100MPa. |  |
| b) | An angle section ISA 75mm x 50mm x 80mm is connected to the gusset plate 10mm thick and is subject to tensile force of 125KN. Design the welded joint taking at= 150Mpa and  = 100Mpa. |  |
| c) | With the aid of neat sketch explain the procedure involved in designing welded connection for member subject to eccentric load producing moment and torque. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a tie member using single unequal angle section to carry an axial load 100KN, connected to gusset plate of 10mm thick with its longer leg. Also design suitable end connection by weld take at =150MPa,  =100MPa. |  |
| b) | An ISMC ISMC200@216.8 N/m is used as a tie member. It is welded to gusset plate of 10mm thick to develop full strength of the member. Design the suitable welded connection by limiting the overlapping length of connection to 300mm maximum. |  |
| c) | Design a tie member using two equal angle sections connected to the same side of gusset plate of 10mm thick to carry an axial load of 200KN. Assume members are connected by sufficient tack welds. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Design a column section adopting two channel sections with an appropriate back to back spacing to carry an axial load of 850KN. The effective length of column is 6m and yield stress of steel is 250MPa. | (8) |
|  | ### OR |  |
| a) | Design a suitable gusseted base for the column consisting of ISHB300@618 N/m with gusset plates of 12mm thick on each flange. The axial load on column is 1200KN. Take permissible bearing pressure on concrete as 400KN/m2. | (8) |
| b) | Distinguish between slab base and gusseted base. | (4) |

…….2/-

-2-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Design a floor beam to carry an udl of 45 KN/m (inclusive of self wt) over an effective span of 6.5m. The depth of beam is restricted to 375mm. If necessary, flange plates may be used. Assume compression flange is laterally restrained. Carryout necessary checks for the beam for shear and deflection. | (8) |
|  | ### OR |  |
| a) | An RC floor is supported on steel beams 4m apart and simply supported over an effective span of 6m. The top chord of beams are encased in the floor slab of 120mm thick. The floor has to support a superimposed load of 4KN/m2. Design the beam and carryout checks for shear and deflection. | (8) |
| b) | Explain, what do you understand by “web buckling” and how do you verify the same. | (4) |
| Q.No.6. | Answer the following Questions: |  |
| a) | Design an angle section purlin for the following data: C/C distance of roof trusses = 4.75m Distance between panel points = 1.35m Load due to roof covering = 140 N/m2 L.L. on roof = 400 N/m2 Wind normal to slopping surface = 1000 N/m2 Slope of roof = 28 | (8) |
|  | ### OR |  |
| a) | Design the principal rafter and main tie for a roof truss using following data: - Principal rafter Panel length = 1.45m Compressive load = 70KN - Main tie Panel length = 1.75m Tensile load = 55KN Adopt double angle equal sections in each of the above case. | (8) |
| b) | Describe how do you consider the effect of wind load and live load in designing a roof truss. | (4) |

***********