## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Automobile Engg.

Subject: Thermal Engineering(4052)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the classification of steam boilers. |  |
| b) | Define Enthalpy, Entropy and Internal energy of steam. |  |
| c) | List the parts in a steam turbine and give their functions. |  |
| d) | State and explain Stefan-Boltzman law. |  |
| e) | Draw an indicator diagram of diesel cycle. |  |
| f) | What are the functions of cooling tower? |  |
| g) | Define: - i) Mechanical efficiency (ii) Volumetric efficiency of an air – compressor. |  |
| h) | Name four refrigerants, and explain any one in brief. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of the Babcock and Wilcox tube boiler showing its principal parts and explain in brief its working. |  |
| b) | What is a water level indicator? Explain with the help of a neat sketch its principle of working. |  |
| c) | What do you understand by the term “Boiler Draught”? Explain in brief “Natural draught” and “Artificial Draught”. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain how quality of steam is determined by using a throttling calorimeter. |  |
| b) | Distinguish between the Impulse turbine and Reaction Turbine. |  |
| c) | Determine the condition of steam in each of the following cases: - At a pressure of 10 bar, if 2640 KJ/Kg are required to produce it from water at 0C - At a pressure of 8 bar and temperature 200C. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with diagram working of two stroke cycle petrol engine. |  |
| b) | A single cylinder SI engine having compression ratio 8:1 running at 2000 rpm develops a torque of 10N-m. The indicated power of the engine is 2.2KW. Find the brake power, frictional power loss and mechanical efficiency. |  |
| c) | With the help of a sketch explain the working of single stage, single acting reciprocating air-compressor. Also draw its indicator diagram. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain heat transfer by convection and differentiate between heat transfer by free convection and forced convection. |  |
| b) | Heat is conducted through a compound plate composed of two parallel plates of different materials A and B of conductivity 134 W/mk and 60 W/mk and each of thickness 36mm and 42mm. If the temperature of outer face of slab A and that of B are found to be steady at 96C and 8C respectively, find the temperature of the interface A/B. |  |
| c) | With the help of a neat sketch explain the working of Parallel–flow type jet condenser. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define a steam-condenser and explain the main two functions of a steam condenser. |  |
| b) | Define: (i) Refrigeration (ii) Air conditioning (iii) Co-efficient of performance (COP). |  |
| c) | Explain under–cooling or sub-cooling with the help of a T-S diagram. |  |

***********