## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Electronics & Drive Control(5102)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give three points of distinction between a SCR & GTO. |  |
| b) | Give a general classification of diodes, stating the basis of their classification. Which of these diodes has least value of reverse recovery time. |  |
| c) | What precaution to is to be taken care of during parallel connection of diodes? Hence draw a neat circuit diagram and explain it in brief. |  |
| d) | How is dv/dt protection provided in thyristors? Explain it in brief. |  |
| e) | Differentiate between the two working principals of 1) ON OFF Control 2) Phase Control used in a.c. voltage controller w.r.t. their circuit diagram & waveforms. |  |
| f) | State the principle of phase controlled converter operation and explain it in brief with the help of neat circuit diagram & waveforms. |  |
| g) | State the advantages of static switches. Also classify a.c. static switches. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a two transistor model of a thyristor. Also state the expression of anode current, clearing indicating each term used in expression. “Once a thyristor starts its conduction it continues to conduct, even if gate signal is withdrawn”. Say True or False and justify your choice. |  |
| b) | What do you mean by commutation in a thyristor? State the method used for commutation of a.c. operated thyristor. Explain it in brief. |  |
| c) | Draw a thyristor firing circuit using UJT and explain its working with the help of waveforms. Give two points of distinction between UJT & PUT. |  |
| d) | In case of a single phase dual converter, draw - Its circuit diagram - Input Waveform - Output waveform. Also state the effect of flow circulating current on its operation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For a three phase full wave bridge types a.c. voltage controller operating resistive load, draw - Its neat circuit diagram - Input voltage waveform - Gate pulses - Output voltage waveform - Output current waveform. |  |
| b) | State the method used to generate 25Hz frequency at the output for the same voltage level as its input, if 50Hz frequency is applied at its input. Draw its neat circuit diagram as well as its input voltage & output voltage waveform & output current waveform. | ……2/- |
|  | -2- |  |
| c) | Draw a neat circuit diagram of a three phase bridge type uncontrolled rectifier and show its input voltage waveform as well as output voltage and output current waveform. Also state the expression for its average output voltage, indicating each term used in it. |  |
| d) | Draw a neat circuit diagram of A.C. Voltage controller with PWM control. Also draw its output current & output voltage waveforms. Also state its advantages. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State the principles of operations of a Chopper. Explain any one of such principle with the help of neat circuit diagram and waveform. |  |
| b) | Explain in brief the working of SCR based series Inverter with the help of neat circuit diagram and load current, voltage across capacitor & voltage across inductor waveforms. |  |
| c) | Classify D.C type static switches and explain in brief the switch used for high power application with the help of neat circuit diagram. |  |
| d) | Classify the switched mode regulator and explain any one of such regulator of your choice with the help of neat circuit diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify d.c. drives. How do you control the speed of d.c. motor by varying the supply voltage using thyristorised devices. Explain in brief such method used with the help of neat circuit diagram. |  |
| b) | How do you control the speed of D.C. drive using armature voltage control with the help of thyristorised devices. Explain the method used with the help of a neat circuit diagram. Also state the expression for armature voltage appearing across o/p of device used. |  |
| c) | Draw a neat circuit diagram of a 1 full wave converter d.c. drive and explain its operation in brief with the help of appropriate waveforms. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the methods used for speed control of a 1 Induction motor. Explain any one of such method of your choice with the help of neat circuit diagram. |  |
| b) | How can you use frequency control method for speed control of 3 phase Induction motor? Can we vary frequency independently without varying the supply voltage applied? Give your choice & justify it. Hence explain the method used in brief with the help of neat circuit diagram. |  |
| c) | How can you control the speed of 3 phase slip ring Induction motor using rotor resistance control with the help of thyristorised device. Explain the method used in brief with the help of neat circuit diagram. |  |

***********